Exposure - Getting it right

By 'exposure' we mean the amount of light that falls onto the film, or CCD if you are using a digital camera. In modern cameras the exposure is usually set to automatic by default and, most of the time, it can be left there and will produce beautiful pictures. There are times though, when the lighting conditions are difficult or we want to produce a particular effect and it would be nice to understand what is going on 'under the hood'.

The problem with all types of film and recording media is that they cannot record the entire range of contrast (black to white) that the eye can see. Especially when you take into account that the eye is constantly adjusting to cope with high contrast. On a sunny day if you look into the shadows of a scene then into the bright areas, the iris in your eye will quickly adjust so you can see detail in both.

Faced with the task of recording as much information as possible, the camera will try to average out all the light levels and expose the film accordingly. As burnt out highlights are normally considered uglier than black shadows, the camera, left to it's own devices will normally err on the dark side. Which is no good if you are shooting someone's face against a bright sky. It's the person's face you want to see, and you don't really care if the sky is white.

## Auto Exposure

The camera manufacturers have come up with all sorts of ingenious metering systems to try to help, there are now multi mode metering systems, which give you a choice of 'centre weighting', 'spot metering' or 'multi spot metering' on many of the better cameras, but none can guarantee to give you what you want every time.

The temptation to think that your camera 'knows what it's doing' is great, even for more experienced photographers, which is my main complaint against automatic cameras. It's easy to get lulled into a false sense of security, switch your brain to more interesting things and end up with a pile of rubbish.

Tip - using auto exposure to your advantage.

If you have a modern camera, the chances are that the default metering system is 'centre weighted average', which means that, although it takes an average reading of the whole scene, it takes more notice of what is in the middle of the frame. Which is good news for us. The other good news is that it takes this reading at the time when you take 'first pressure' on the button to take your picture. When you push it halfway down and it beeps at you, not only is the focus now set (on an auto focus camera) but the exposure reading is taken and the aperture and shutter speed are set. So, if your main point of interest is not in the centre of the frame, it's a good idea to put it there temporarily while you focus and take your light reading, then move the camera whilst still holding the button halfway down and compose the picture the way you want it to be. A common use for this technique is when you are taking a close up shot of two people and there is space between their heads, if you're not careful the camera will focus on the wall or trees behind them. If the background is very dark or very light this can alter the exposure significantly and result in faces that are too dark or too light.

Skin tones are what most meters are set up to consider an 'average tone', they are also usually the part of the picture that we most want to get right. If I am photographing a group of people in difficult circumstances, like bright sunlight for instance, I will often move close in to the group and take a light reading from someone's face or, if we are all standing in the same type of light, I will take a reading from the back of my hand. This is no good, of course, if the subject is in bright sunlight and I am in the shade. Brown parcel paper is also an extraordinarily accurate surface to take a light reading from.

## Manual Exposure

Now it's time to turn that dial away from 'programme' mode and have a look at the dreaded 'manual' mode. There are also a bewildering array of other choices such as 'aperture priority', 'shutter priority', 'exposure compensation' etc., but once you understand the basics you will be able to select the most suitable mode.

The amount of light falling on the film is governed by three things.

- The amount of light reflected from the scene which, if you are outdoors, you can do very little about.

- The 'shutter speed' which is the amount of time the shutter is open, measured in fractions of a second.

- The 'aperture setting' which is the size of the hole through which the light enters. If you look at the lens of your camera you will see a diaphragm in the middle of the glass which the camera adjusts according to the light. This does exactly the same job as the iris in your eye. Aperture settings are measured in 'f stops'. For an explanation of 'f stops' click here.

The shutter speed and aperture settings have other quite separate effects on the photograph which we will discuss in another article, but for the purposes of exposure, making the picture darker or lighter, they are interchangeable. Make the hole twice as big and open the shutter for half the time and you will expose the film the same amount.

## Why use manual exposure?

The advantage of manual exposure is that the settings do not keep changing as your scene changes. Let's suppose that you are taking close up photos of cars passing by. Some of the cars will be black or dark colours and some will be light colours or white. If you are filling the frame with almost nothing but car, the meter will be trying to render each car as mid grey. Although it will probably not succeed, what you will notice is that the background is a different shade in each photo.

I often have to take portraits of people, some are wearing very dark clothes and some are wearing white. If I am not careful with my light readings the skin tones will be affected by the clothes.

Although it is by no means always necessary to use manual exposure, an understanding of how it all works will save a lot of disappointment.

# Shutter Speed and Aperture

## So what's the difference between shutter speed and aperture?

## Shutter Speed

Although, as discussed in exposure, the shutter speeds and apertures are interchangeable as far as exposure is concerned, they each have their own unique effect on the picture. Let's take a look at shutter speeds first as their effect is easily understood.

The shorter the time that the shutter is open the sharper the photo will be.

If you are photographing fast moving objects such as cars or people running you need to select fast shutter speeds to capture the sharpest picture you can. One exception to this is when you are panning the camera with the subject, the object of the exercise here is to render the subject sharply and blur the background, so a careful selection of the right shutter speed to do both is necessary. I often find that a little blur in the right places on a picture gives a greater sense of movement than if everything is pin sharp. This blur, however, must be in the right places, normally we want to see the head and torso rendered sharply but, if the feet and hands are blurred, it can often be a good thing. Blurring the background can also get you out of trouble when there is a lot of clutter that will detract from the main subject. Getting the shutter speed right to render the correct balance of sharpness and blur on any given subject can really only be determined through trial and error. One of the great advantages of the digital camera with it's instant playback is that this learning process can be a lot shorter than it was before. If you have a zoom facility on your playback of pictures, now is the time to get familiar with it. I had my digital camera for quite a while before I realised that I could review my pictures and zoom in to check the sharpness.

Not only moving objects suffer from too slow a shutter speed. If you are holding the camera in your hand rather than having it mounted on a tripod, you will see the telltale signs of 'camera shake' (i.e. the movement of the camera) at shutter speeds longer than 1/125th of a second. A secure pair of hands will be able to get away with 1/60th or even 1/30th of a second but the camera would be better mounted on a tripod. Once again I will say at this point that the difference between a mistake and an effect is usually the degree. A small amount of blur would be considered a mistake, whereas really blurred streaks of light can be an interesting effect. It's all a question of convincing the viewer that you intended to do it.

Tip - When the shutter speed is important as with moving objects, it's a good idea to set the camera to 'Shutter Speed Priority' mode. This is where you select the shutter speed and the camera selects the appropriate aperture according to the light reading.

Of course, if you are taking photos of static objects like houses with a camera mounted on a sturdy tripod, you can leave the shutter open as long as you want without blurring. An interesting by-product of this, if you get to see really old photos taken in the first part of the 19th century, you will see that there are almost no people in the photos at all. That is because the exposure times were so long that the people had walked through the scene without being rendered. For the same reason the really early pictures, in the time of Niépce, the late 1830's, have almost no shadows because the plates took all day to expose and the sun moved across the sky illuminating the scene from both sides.

## Apertures

As well as letting more or less light into the camera the size of the aperture you choose governs the 'Depth of Field'. Depth of field means the amount of the picture, from foreground to background, that is in sharp focus. A smaller aperture will give you a greater depth of field and a larger aperture will give you a more restricted depth of field. This characteristic can be used to good effect in many ways.

If you are photographing vast landscapes on a sunny day, the chances are that everything will be in focus and you will not notice this phenomenon at all. Depth of field, or the lack of it, is much more noticeable when taking close-ups. As I mentioned in the section on moving subjects, it is often desirable to render the background of your picture out of focus. This is easy to achieve by selecting a larger aperture to restrict the depth of field.

|  | Conversely, when photographing very small objects (as in the picture opposite) getting everything in focus can be quite a challenge and may require a very slow shutter speed in order to be able to use the smallest aperture available. The focal length of the lens makes a difference to the depth of field available, the longer the lens the more restricted the depth of field. A wide angle lens will give you almost limitless depth of field. Tip - If depth of field is important to either make sure everything is in focus or to throw some things out of focus, select the 'Aperture Priority' mode on your camera. In this mode you select the aperture and the camera selects the shutter speed according to the available light. Tip - If you are shooting in bright light and want to restrict the depth of field, use a neutral density filter in front of the lens to reduce the light entering the lens. These are available in different densities, 2x, 4x, 8x etc. each one cutting the light in half, quarter, eighth etc. In extreme circumstances you can screw a couple of them together. Although they are 'neutral density' filters and should not effect the colour balance, if you use two or more together you might need a little colour correction at the printing stage. |
| --- | --- |

The Shutter and the Aperture.

Of the various controls on your camera the aperture and shutter controls are the ones which are going to give a lot of control over the content of your finished photographs. It is important to understand from the outset what they are each responsible for and how they affect each other. We have already looked at how the aperture and shutter affect exposure and thier relationship in that context. As you, hopefully, know you can use many combinations of shutter and aperture yet still retain the same exposure value, so we are now going to take a look at the factors which will influence how you will select a particular combination of shutter and aperture.

In a nutshell, the shutter controls movement, which can be subject movement or camera movement (shake), and the aperture controls how much of the scence (from front to back) will be in sharp focus. This area of sharpness is known as the 'depth of field'. Depth of field is actually influenced by two factors: Aperture and focused distance.

Juggling.
Much of the use of aperture and shutter is juggling one with the other. If you want a lot of depth of field you will have to select a small aperture. To counter this you will have to select a shutter speed that will;

- give you the correct exposure.

- be sufficiently fast enough to freeze movement within the scene.

- be fast enough to prevent camera shake.

If you want to capture fast movment you will have to select a reasonably fast shutter speed. To counter this you will have to select an aperture which will;

- give you the correct exposure.

- be small enough provide sufficient depth of field.

There will be many occasions where you will have to make compromises, particularly if you are hand holding your camera. Shallow depth of field is fine so long as the most important part of the suject is sharply focused and some subject movement may also be acceptable but camera shake will consign just about anything to the bin. Unless , of course, the picture is very newsworthy or is of personal significance.
We will go on to look at the aperture shortly but first we will look at the shutter, shutter speeds and the dreaded camera shake. It happens to everyone at some point and you can't say 'I don't know whats wrong , this has never happened to me before'.

Camera Shake.

When you take a photograph you capture a moment in time. The faster the shutter speed is the shorter the moment will be. If you use a fast enough shutter speed the moment will appear frozen and without movement. When you are choosing a shutter speed you will have to take into account two separate types of movement:

- subject movement. This is anything within the picture area that is moving or may move during exposure.

- camera movement or 'camera shake'. This is movement of the camera during exposure.

Movement appears as blurring of the image. Subject movement will only affect the part or parts that actually move during exposure while camera shake will affect the whole image to the same degree.
Before we look at how to choose shutter speeds we will address what is a common cause of ruined photographs.
Camera Shake.
Unlike subject movement which is relative to the camera, and you can often see it, camera shake is movement of the camera relative to the subject and, as it happens during exposure, you cannot see it. Everyone suffers from camera shake at some time or other.
Most instances of camera shake occur during hand held photography. This, as you have probably noticed, is a very popular way to take photographs so it is wise to know when you are likely to fall victim to camera shake. Like using a fast enough shutter speed to freeze movement in a photograph you must also use a fast enough shutter speed to freeze any movement of the camera. The actual shutter speed you use to combat camera shake is generally determined by a couple of factors.

- an individuals ability to hold a camera steady.

- the focal length of the lens in use at the time.

- magnification. If you are shooting close-ups camera movement is magnified along with your subject.

| Many people over estimate how steady they can hold a camera but minimum shutter speeds for your average Joe Punter hand holding a camera are usually given as the inverse of the focal length in use. The word 'inverse' comes from the world of mathematics and as such is a bit scary to a lot of people but in this case it just means 'one over'. As in 'one over the focal length'. Here is a wee example of how easy it really is. Minimum recommended shutter speeds for hand held photography. |  |
| --- | --- |
| 24mm | 1/30th |
| 28mm | 1/30th |
| 50mm | 1/60th |
| 70mm | 1/60th |
| 90mm | 1/125th |
| 135mm | 1/125th |
| 200mm | 1/250th |
| 300mm | 1/250th |
| 500mm | 1/500th |

Using a standard 50mm lens the minimum shutter speed you should use will be 'one over 50', which is 1/50. To turn it into a shutter speed you simply call it a fraction of a second, like this; 1/50th of a second. The nearest shutter speed you can set is actually 1/60th of a second but you get the idea.

Or.

Using a 200mm lens the minimum shutter speed you should use will be 'one over 200', which is 1/200th of a second. So you would set your shutter speed to 1/250th of a second, that being the closest to 1/200.

These figures are just a rough guide and may vary according to individual ability. Over time you will learn how slow a shutter speed you can hand hold but if in doubt always err on the fast side. It would be unwise to hand hold any camera/lens combination below 1/30th of a second.

Tip:  Try slightly pulling the camera while taking the picture.  This will frequently help you hold the camera still and prevent camera shake.

Choosing and Using Shutter Speeds.

Your choice of shutter speed will have a controlling effect on how movement will be captured in the finished photograph. Too slow a shutter speed will result in blurring of the subject while too fast a shutter speed can sometimes cause a scene to look unnaturally static. In choosing a shutter speed you will have to consider several factors.

- camera movement.

- subject movement.

- how much movement you want to show.

- aperture setting.

Subject movement.

For most people the most important aspect of choosing a shutter speed is getting one fast enough to freeze movement. Unfortunately the world is full of things that all move at different speeds and no one bothered to make a list of what shutter speed would be needed to freeze each and every one of them. With this in mind I have put together a little table containing some common 'things that move' and a suitable shutter speed to stop them. You can use these values as a basis for working out shutter speeds for other situations you may encounter This is a rough guide only and there is no substitute for practice.

| Still Life | Shutter speed will not affect a subject which is completely static |
| --- | --- |
| Landscape | Landscapes are fairly static but wind will move clouds, grass, leaves etc 1/30th of a second if you want it still. Less if you want a bit of life in it. |
| Street scene. | 1/60th - 1/125th of a second. |
| Your holiday photos. | 1/60th - 1/125th of a second. |
| Person walking. | 1/125th - 1/250th of a second. |
| Person running. | 1/500th - 1/1000th of a second. |
| American football, soccer, rugby etc. | 1/250th - 1/1000th of a second and lots of practice. |
| Car, approx. 30 mph. | 1/1000th -1/2000th of a second. |
| Motocross. | 1/500th of a second and over and lots of practice. |
| F1 / Indy car. | 1/1000th of a second and over plus lots of practice. |

It is probably worth mentioning that subject movement is also relative to the distance from the camera. That means something moving at a fixed speed will appear to move faster the closer it is to the camera / larger in the viewfinder. On top of that, movement is more apparent travelling across your field of view than it is moving towards you.

# Night Photography

Night photography has an attraction all its own. There's something about scintillating lights from office windows hanging in the dark of the night -- a modern version of the starry skies -- that appeal to us. Whether it's a city skyline, lamp posts on a dark and deserted street, or the front of your house all decked out with holiday lights, the challenge of capturing the mood of a night scene depends on whether your digital camera is capable of night photography and on a couple of simple techniques.

Can My Camera Do Night Photography?

For successful night photography you need a digital camera that allows you to keep the shutter open for a long time, anywhere from 3 to 30 seconds. Check your camera specifications in the User's Manual under Shutter Speed. The shutter speeds available will be given as a range, e.g. 30 sec. - 1/2,000 sec.

If you are seriously into night photography, then you would want to ensure you purchase a digital camera that allows the longest shutter speed possible, and even Bulb (where the shutter remains open as long as you depress the shutter release button).

But before you plunk down your money for that digital camera, there's two more features to verify -- and one accessory to purchase, if you don't have it already.

Shooting Modes

For an image to be captured by a digital camera's image sensor, the latter requires exposure to light. But at night, light is what we don't have enough of.

Some of you may have noticed that, if you select a shooting mode of Auto (A) or Program Auto (P), your night pictures always come out too dark. They are simply underexposed. But, why is that -- if your camera's shutter speed ranges from, say 10 sec. to 1/2,000 sec.?

Go back to your camera's User's Manual and look a bit more carefully. Are all the shutter speeds available in Auto or P mode? Ah-ha, many digital cameras (we're talking consumer models here) do not make the whole shutter speed range available in A and P mode! Perhaps the slowest shutter speed available in A and P mode is only as slow as 1/3 sec. That's usually not long enough for night photography. To access the longer shutter speeds, you may need to select one of the other shooting modes, e.g. Shutter-Priority, or even switch to full Manual mode.

So ensure that your digital camera has full Manual mode and allows access to the full range of slow shutter speeds in that mode.

Self-Timer & Remote Controller

Another feature that you want your digital camera to have is a self-timer or, ideally, a remote controller. The purpose is to allow you to depress the shutter release button without introducing camera shake. I particularly like the remote controller, but not every camera comes with one or even has one available optionally.

But almost all, if not all, cameras has a self-timer. Usually the self-timer counts down from 10 sec. I find that a bit long to wait, especially since you would need to take more than one shot and it's minus 10 with the wind chill outside. The cameras that additionally provide a 2 sec. self-timer have my nod of approval here.

Tripod

A mandatory accessory that you need is a sturdy tripod. When you let the shutter stay open for a long time, the camera needs to be kept rock steady, otherwise you end up with blurred images.

Examples

OK, so we have our digital camera and tripod, and are ready to venture forth into the night in search for interesting night shots. When we find one, we set up camera and tripod, frame and... what do we do now?

Well, the images below show what happens when you take the same shot using P mode, then in Manual mode with various different shutter speed/aperture combinations, all in search of the correct exposure. The camera was on a tripod for all three shots.

Programmed Auto Mode - Underexposed

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 1/4 sec., Aperture F2.8, ISO 80

In the above example, the camera uses the slowest shutter speed and largest aperture available in P mode and at the widest focal length. The picture is underexposed.

Manual Mode - Overexposed

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 3 sec., Aperture F2.8, ISO 80

Switching to Manual mode allows me to access the slowest shutter speed available on this camera, 3 sec. while keeping the aperture at F2.8 (the largest aperture available). The effect is immediately better, but it does seem a bit too bright, giving almost a daylight effect. If this is the effect you're after, then you're done. But if you wanted to capture the night mood, read on.

Now it is just a matter of adjusting the shutter speed and/or aperture to obtain the desired exposure. I choose to close down the aperture so as to increase the depth of field also.

Manual Mode - Correct Exposure

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 3 sec., Aperture F4.0, ISO 80

Closing down the aperture to F4.0, a more pleasant image is obtained with enough dark areas to indicate it is night time (dusk, really) and enough lighted areas to reproduce what my eyes saw at the outdoors skating rink at the Mississauga Civic Center.

ISO

For the three pictures above, I used ISO 80, the lowest ISO available for best image quality. But what if at 3 sec. and F2.8 (i.e. at max. exposure possible for this particular camera), the image still came out too dark? In this case, I would need to increase the sensitivity of the image sensor to a higher ISO. Do note that increasing the ISO also increases the amount of noise visible in your images.

Technique

Take a number of shots at different shutter speed/aperture combinations.

Immediately review the shot as soon as you've taken it.

Ensure your LCD brightness is set to Normal, not Bright, for a truer representation of your recorded image.

A good aperture to start with is F4.0 or F5.6 (for greatest depth of field), and adjust shutter speed up or down until you're satisfied with the shot.

For good measure, take an extra shot past your optimum exposure setting. For example, if you were progressively using longer shutter speeds, and you think you've find the correct one, take an extra shot with the next longer shutter speed. Conversely, if you were using progressively faster shutter speeds, take an extra shot using the next faster shutter speed.