- What Is... Aperture?

When you look at the technical specifications of a digital camera, one of the very first specification mentioned is its maximum aperture and/or its aperture range.

What is 'aperture,' what is a good aperture range, and how is aperture relevant when it comes to choosing a digital camera?

What Is Aperture?

The main function of a camera lens is to collect light. The aperture of a lens is the diameter of the lens opening and is usually controlled by an iris.The larger the diameter of the aperture, the more light reaches the film / image sensor.

Aperture is expressed as F-stop, e.g. F2.8 or f/2.8. The smaller the F-stop number (or f/value), the larger the lens opening (aperture).

[Note: Many camera user manuals today will refer to the aperture in terms of "aperture value" instead of f/value. I'm not sure when this trend started but don't get confused between "aperture" and "aperture value." Aperture value" is simply another way of saying f/value.]

In practice, unless you are dealing with a fixed-aperture lens (many simple point-and-shoot cameras have only one fixed aperture), the aperture of a lens is usually expressed as a range of fstops.

When you read the specifications of a camera, the aperture may be expressed in a number of different ways, the following three being the most common:

- Maximum Aperture:

|  | Max. Aperture | F2.8 |  |
| --- | --- | --- | --- |

This simply states that the maximum aperture for the lens is F2.8.

- Aperture Range:

|  | Aperture Range | F2.8-F8.0 |  |
| --- | --- | --- | --- |

This states the max. and min. aperture, the assumption being that there are standard increments between them.

- Maximum Wide-Angle and Telephoto Apertures:

|  | Aperture | F2.8(W)-F3.5(T) |  |
| --- | --- | --- | --- |

This gives the max. aperture for the wide-angle (F2.8) and telephoto (F3.5) focal lengths of a zoom lens.

A "fast" lens is one that has a large maximum aperture (F2.4, F2.0 for current digital cameras; F1.4, F1.2 for 35mm film cameras).

Quick Quiz: which lens has a larger opening (aperture): one with an aperture of F1.8 or one with an aperture of F2.8?

Answer: F1.8 (remember, the smaller the F-stop, the larger the aperture)

A Good Aperture Range

My personal preference for a 'good' aperture range is:

F1.8 - F16

| F1.8 | F2.8 | F4 | F5.6 | F8 | F11 | F16 |
| --- | --- | --- | --- | --- | --- | --- |

This tells us that the camera has an aperture range of F1.8 to F16; the maximum aperture is F1.8, and the minimum aperture is F16.

There are 5 f-stops between the max and min aperture. If your camera's lens is currently set at an aperture of F5.6, closing it by 1 f-stop would mean selecting F8; opening it up by 1 f-stop would mean selecting F4.

| F1.8 | F2.8 | F4 | F5.6 | F8 | F11 | F16 |
| --- | --- | --- | --- | --- | --- | --- |

How Is A Large Maximum Aperture Relevant?

A large maximum aperture is preferable to a smaller one since it gives the photographer more latitude in the kind of pictures that can be taken.

For example, it is pretty obvious that the larger the aperture, the better your digital camera will perform in low-light situations, since a larger lens opening is able to admit more light than a smaller lens opening.

A larger max. aperture also allows you to use a faster shutter speed to freeze action.

So, let's say the light meter in your digital camera calculates that for proper exposure in that indoor arena, you need an aperture of F4 and a shutter speed of 1/60 sec.

F4

1/60

To use a faster shutter speed (say, 1/250 sec.) to freeze action, you have to open up the aperture to allow more light in for that shorter amount of time.

For every shutter speed increment we go up, we need to open up a f-stop of aperture. From 1/60 sec. to 1/250 sec. there are 2 increments, so we open up the aperture by 2 f-stops, going from F4 to F1.8. Note that the camera would give proper exposure at 1/60 sec. at F4, 1/125 sec. at F2.8, and 1/250 sec. at F1.8, since all three aperture/shutter speed combinations allow the same amount of light into the camera. [Some digital cameras provide a 'Program Shift' function to allow that very shifting of aperture/shutter speed combinations in tandem.]

| F1.8 | F2.8 | F4 | F5.6 | F8 | F11 | F16 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/250 | 1/125 | 1/60 | 1/30 | 1/15 | 1/8 | 1/4 |

Of course, in a digital camera set on Auto mode, you can select Sports scene mode, and the camera will automotically select a fast shutter speed and the appropriate aperture. Likewise, in Shutter-Priority mode, you can choose which shutter speed you want (fast or slow), and the camera will select the appropriate aperture for proper exposure.

In our example above, let's say the lens on your digital camera only opens up to a max. aperture of F2.8. If you now select 1/250 sec. (in Shutter-Priority mode), the camera will not be able to select an aperture larger than F2.8 (in our example, it really needs F1.8). It would then give you an "underexposure" warning. If you go ahead and take the picture anyway, your picture would be 1 f-stop underexposed (i.e. you really needed to open up the aperture by 1 more f-stop for correct exposure).

Similarly, if you select a shutter speed of 1/4 sec. and the lens only closes down to a min. aperture of F8 (in our example, it really needs F16), the camera would give you an "overexposure" warning. If you go ahead and take a picture anyway, your picture would be 2 f-stops overexposed (i.e. you really needed to close down the aperture by 2 more f-stops for correct exposure).

[Editor's note: There is a third variable in the above example which we have purposefully not introduced. This is the sensitivity of the image sensor -- the ISO. We'll cover this in a later tutorial.]

How Is A Small Minimum Aperture Relevant?

A small minimum aperture is preferable to a larger one since it also gives the photographer more latitude in the kind of pictures that can be taken.

Suppose we want to take a picture of flowing water. As mentioned above, to depict flowing water, we usually want to use a slow shutter speed so that the water blurs. It is this blurring that makes the picture so effective in depicting water motion.

So, let's say the light meter in your digital camera calculates that for proper exposure on a bright sunny day, you need an aperture of F8 and a shutter speed of 1/125 sec.

F8

1/125

Well, if you decide to use a slower shutter speed (say, 1/30 sec.), this means that you have to compensate by closing down the aperture to allow less light in.

It makes sense really. Since you have increased the time the shutter remains open to allow light in, you must compensate by allowing less light in to expose the image sensor in that longer amount of time, if you still want a properly exposed picture.

But, what if the lens on your digital camera closes to a minimum of F8? You're stuck at the shutter speed of 1/125 sec. If you use 1/30 sec. at F8, your picture will be overexposed, i.e. burnt out. At 1/125 sec. and F8, your picture will be properly exposed but the fast shutter speed will freeze the water motion and you won't obtain the blurring effect you desire.

If the lens in your digital camera closed down to F16, presto, your problem is solved! 1/125 sec. at F8 is equivalent to 1/30 sec. at F16, which means that you would have a perfectly exposed purposefully-blurred-for-effect flowing water shot. [If your lens does not close down to F16, you could use a Neutral Density (ND) filter to reduce the amount of light coming into the lens, and thus allowing you to use a slower shutter speed.]

| F1.8 | F2.8 | F4 | F5.6 | F8 | F11 | F16 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/2,000 | 1/1,000 | 1/500 | 1/250 | 1/125 | 1/60 | 1/30 |

[Editor's note: There is a third variable in the above example which we have purposefully not introduced. This is the sensitivity of the image sensor -- the ISO. We'll cover this in a later tutorial.]

Aperture and Depth of Field (DOF)

We mentioned this above but feel it is important enough to repeat in its own paragraph.

The Depth of Field is the distance wherein objects are in focus.

There are times when you desire a great depth of field, i.e. where objects both close to you and far from you are in focus. This is especially true when you are taking a landscape picture and want as much as possible to be in crisp focus.

Then there are times when you want to isolate your subject, as when you are taking a portrait and want your subject to be in sharp focus but the background to be out of focus. In this case, you desire a shallow depth of field.

One way to influence DOF is by selecting the appropriate aperture.

The rule of thumb is this:

- Select a large aperture (or small f/value or small aperture value), e.g. f/2.8, to obtain a shallow DOF

- Select a small aperture (or large f/value or large aperture value), e.g. f/8.0, to achieve great DOF

Here are some real images that demonstrate the use of aperture to influence DOF:

| Aperture and Depth of Field (DOF) AF area is on yellow pistils of flower in front |  |
| --- | --- |
| Shallow DOF | Great DOF |
| Note how the use of a large aperture (small aperture value) throws the flowers in the background out of focus. Focus has to be precise. | Using a small aperture (large aperture value) extends the DOF from the foreground all the way to the background. |
| 49.8 mm, Av, Spot, 1/30 sec., f/3.5, +0.7EV, Macro, Tripod used | 49.6mm, Av, Spot, 1/5 sec., f/11, +0.7EV, Macro, Tripod used |

Note: Since we are on the subject of DOF, DOF also changes with focal length. Use a small focal length to increase DOF, a longer focal length obtain a shallower DOF. I.E. if you zoom, the DOF decreases. [OK, to be technically correct, focal length does not really affect DOF, but gives the effect that it does. And, that's good enough for us since that's what we're after. We'll cover DOF in a later tutorial.]

Due to the small image sensors used, digital cameras use such small focal lengths that it is very difficult to obtain shallow depth of field even when using a large aperture. In the example above, we used a large aperture AND a long focal length (telephoto macro) to achieve a shallow DOF.

Recap

A large maximum aperture is a good thing. It allows more light to reach the image sensor, and so allows you to use a faster shutter speed. A faster shutter speed freezes action and negates the effect of camera shake, resulting in pictures that are not blurred.

Another advantage of a large maximum aperture is to provide a shallow depth of field. This allows the background to blur nicely thus isolating your subject (especially effective when taking portraits).

A small minimum aperture is also a good thing. It allows you to use a slow shutter speed on a bright sunny day. A slow shutter speed allows you to depict motion.

Another advantage of a small minimum aperture is to increase the depth-of-field. An increased depth-of-field allows you to take landscape pictures where as much of the picture in the foreground and reaching all the way to the background (usually, 'infinity') is in sharp focus.

# What Is... Exposure Bracketing

Exposure bracketing is a simple technique professional photographers use to ensure they properly expose their pictures, especially in challenging lighting situations.

When you expose for a scene, your camera's light meter will select an aperture / shutter speed combination that it believes will give a properly exposed picture.

Exposure bracketing means that you take two more pictures: one slightly under-exposed (usually by dialing in a negative exposure compensation, say -1/3EV), and the second one slightly over-exposed (usually by dialing in a positive exposure compensation, say +1/3EV), again according to your camera's light meter.

The reason you do this is because the camera might have been 'deceived' by the light (too much or too little) available and your main subject may be over- or under-exposed. By taking these three shots, you are making sure that if this were ever the case, then you would have properly compensated for it.

As an example, say you are taking a scene where there is an abundance of light around your main subject (for example, at the beach on a sunny day, or surrounded by snow). In this case, using Weighted-Average metering, your camera might be 'deceived' by the abundance of light and expose for it by closing down the aperture and/or using a faster shuter speed (assuming ISO is constant), with the result that the main subject might be under-exposed. By taking an extra shot at a slight over-exposure, you would in fact be over-exposing the surroundings, but properly exposing the main subject.

Another example would be the case where the surrounding might be too dark, and the camera exposes for the lack of light by either opening up the aperture and/or using a slower shutter speed (assuming ISO is constant), then the main subject might be over-exposed. By taking an extra shot at a slight under-exposure, you would in fact be under-exposing the surroundings, but properly exposing the main subject.

Now, most digital cameras have auto exposure bracketing (AEB), meaning that if you select that option before taking your shot, the camera will automatically take three shots for you: one which it thinks it has perfectly exposed; a second one sightly under-exposed; and the third one slightly over-exposed. The amount of under- and over-exposure usually defaults to -1/3EV and +1/3EV, but can also sometimes to specified in SETUP, e.g. you may want to use -1EV and +1EV instead.

When should you use exposure bracketing? Anytime you feel the scene is a challenging one (too much highlights or shadows) as far as lighting is concerned -- e.g. sunsets are usually better taken slightly under-exposed so use exposure bracketing there -- or whenever you want to be sure you don't improperly expose a fabulous shot that you may not get the chance to go back and take again.

Remember, you are not using film anymore, so there are really no wasted shots (unless you are severely constrained by the size of your storage media).

Digital Dodging & Burning

Should you delete the extra shots right away? No, if storage permits, keep all three shots until you get home and upload them to your PC and into an image editing software, such as Photoshop. By using the layers feature of Photoshop (or similar feature of another image editing software), you can load all three shots into different layers and then carefully erase the under-exposed or over-exposed part of one or more layers to end up with a final shot where both the main subject and the surroundings are properly exposed!

With this layers feature in mind, you may now fearlessly shoot in very extreme lighting situations where there are many parts in different intensity of light and shadows such that you are losing details in the highlights and shadows. In this case, you might need more than two extra shots to obtain details in the different parts. Without moving the camera (a tripod is essential here), take as many shots as you need, exposing for the different parts you want details to be visible. Then you would load them all up into Photoshop, each into its own layer, and by erasing the under- and over-exposed parts in each layer (granted, this equivalent of film 'dodging' and 'burning' can be a very tedious and challenging task in itself, but done properly it can be well worth the effort), you can end up with an 'impossible' shot where every part of the cave is properly exposed.

In a studio setting, again with camera on tripod, you move a portable studio light around to different parts of a subject (e.g. a car) and work your magic in your image editing software to produce an image that is "impossibly" perfectly exposed all around.

Used judiciously, exposure bracketing is a simple technique that can ensure proper exposure of a difficult lighting situation.

To read more about this technique (also called 'Blended Exposures'), visit Max Lyons Digital Image Gallery.

RAW File Format

Many digital cameras targeted to advanced amateur photographers provide a RAW file format. This option allows you to save your image data as nearly identical as it is captured by the image sensor -- theoretically with no processing applied (though in practice, a minimal amount of necessary processing may be applied, depending on the algorithm used).

It is equivalent to a digital negative and allows the photographer to manipulate this RAW image data after the fact in an image editing software (with the appropriate RAW plug-in) to produce a picture that is correctly exposed and with faithful colour reproduction.

If your camera provides that option, you may want to experiment a bit -- you'll see that it pretty much allows you to perform some exposure bracketing after the fact.

Some professional photographers only takes pictures in RAW while others don't. It all depends on how much control over the final image they need. The downside of using RAW: saving images in the RAW file format takes much more memory space and takes longer to write to memory -- plus you need to process every single image.

- What Is... Correct Exposure?

When you take a picture with your camera set on Auto mode, you are delegating responsibility for determining the correct exposure to the camera. Depending on the 'brain' (or programmed chip) inside your camera, the result may be pleasing or not to your satisfaction. But before you blame the camera for your lousy pictures, it pays to understand a bit what goes on behind the scenes when you press the shutter release button. In this article, we are going to look at what 'correct exposure' means.

Aperture and Shutter Speed

A correctly exposed image means that the right amount of light has exposed the image sensor. There are basically two ways your camera can ensure that: 1) open or close the aperture (by making the hole of the iris larger or smaller; or, as is becoming more and more common in point-and-shoot digicams, by using a neutral density filter to restrict the amount of light reaching the image sensor); 2) by deciding how long to leave the shutter open; 3) by adjusting the ISO.

| F2.8 | F4 | F5.6 | F8 |
| --- | --- | --- | --- |

When you (or your camera) uses a larger aperture (e.g. F2.8), the hole of the iris is larger and more light reaches the image sensor. Conversely, using a smaller aperture (e.g. F8) means that the hole of the iris is smaller and less light reaches the image sensor.

When a fast shutter speed is used (e.g. 1/1,000 sec.), the image sensor is exposed for only that small amount of time (i.e. 1/1,000 sec.). Conversely, when a slow shutter speed is used (e.g. 1/30 sec.; some cameras allow slow shutter speeds up to 30 sec. or more), the image sensor is exposed for that longer amount of time.

F8

1/30

Obtaining correct exposure is a setting combination of aperture and shutter speed. For example, your camera's light meter may have measured a need for an aperture of F8 at a shutter speed of 1/30 sec. If you press the shutter release button now, your camera will take a correctly exposed picture.

Complications

However, here is where the complications show up. If you are taking a picture of a serene and calm landscape, a setting of F8 at 1/30 sec. may be perfect! The small aperture gives you good depth of field ensuring that objects near and far are in focus. The slow shutter speed may be a problem if you are not using a tripod. A slow shutter speed means that any camera shake (even ever so slightly) will result in some blurring of the final image. A perfectly exposed blurred image!

Two things you've learned right here: 1) a small aperture increases depth of field; 2) a slow shutter speed requires a tripod, or other ways to hold the camera steady (e.g. by bracing yourself against something).

But, what if you are trying to take a picture of your son flying a kite in the park? A slow shutter speed will not only result in a blurred image because of camera shake, but your son is moving fast across the camera and the resulting image is a perfectly exposed blurred image of your son. To 'freeze' fast action, you need to use a fast shutter speed. So, move the scene mode dial to 'Sports/Action' mode or, if your camera allows Shutter Priority mode, select that mode and set a shutter speed of 1/250 sec. To still obtain correct exposure, you also need to open up the aperture now.

Remember, you selected a faster shutter speed which means that the image sensor will be exposed for a shorter time. If you don't open up the aperture but keep F8, then the image sensor receives less light (you maintained the same hole opening, but closed the shutter sooner), and the resulting image will be underexposed. In this case, you would have 'frozen' the action, but unexposed the image.

Shifting Aperture and Shutter Speed

| F2.8 | F4 | F5.6 | F8 |
| --- | --- | --- | --- |
| 1/250 | 1/125 | 1/60 | 1/30 |

Note that it is also possible to shift the aperture/shutter speed and still obtain correct exposure. As is illustrated above, if 1/30 sec. at F8 gives a correct exposure, then the three other aperture/shutter speed combinations shown also give a correct exposure. It is easy to understand when you consider that each aperture shift from F2.8 to F8 halves the amount of light reaching the image sensor. To maintain correct exposure, you therefore have to double the time the shutter remains open.

The Garden Hose Metaphor

Let's take an aside here to explain a bit more about aperture/shutter speed combination. Think of your garden water hose that you are using to fill a bucket with water. The diameter of the hose can be thought of as the aperture: the larger the diameter, the more water flows through. The length of time you leave the tap open can be thought of as the shutter speed: the longer you leave the tap open, the more water flows through. The speed of water flow can be thought of as the ISO: the faster the water flows through the hose, the more water flows through. The amount of water that collects into the bucket is the exposure.

Let's also pretend that you have two water hoses, one with a small diameter (our F8 aperture) and the other one with a larger diameter (aperture of F2.8).

To fill our bucket, we experiment with the smaller of the two hoses and find that we need to leave the tap open for 10 min (our shutter speed of 1/30 sec.).

So, here we have the following 'exposure setting':

- small hose & long time of 10 min.

- small aperture (F8) & slow shutter speed of 1/30 sec.

OK, so what happens if you use the same small hose but close the tap after, let's say, 3 min? Of course, it's clear that the bucket won't be full.

That is exactly what you did when you kept the same aperture and used a faster shutter speed. Not enough light came in to properly expose the image sensor ('the bucket is not full').

What do you have to do to fill up the bucket? Use the bigger hose! Aha, now with more water gushing out of your bigger hose, you can now close the tap earlier and still obtain a full bucket.

Similarly, to use a faster shutter speed, you need to use a larger aperture. Dial in F2.8 at 1/250 sec. and presto! you've just taken a perfectly exposed picture of your son frozen in motion.

Here is your new 'exposure setting':

- big hose & short time of 3 min

- large aperture (F2.8) & fast shutter speed of 1/250 sec.

Two more things you've learned right here: 1) a fast shutter speed will 'freeze' action; 2) the combination of shutter speed/aperture walks in opposite direction to each other: for a particular shutter speed/aperture combination required for correct exposure, if you now increase the shutter speed, you also need to open up the aperture; and if you use an even slower shutter speed, you will need to use an even smaller aperture.

Of course, depending on the specifications of your digital camera, you may have only a restricted set of aperture/shutter speed combinations. For example, a common shutter speed range is 1 sec - 1/1,000 sec. and a common aperture range is F2.8 - F8.

That is the range of aperture/shutter speed you can play with to obtain correct exposure. If you can't select a combination for correct exposure using these ranges, then your image will be either over or underexposed.

Over and Underexposure

| F1.8 | F2.8 | F4 | F5.6 | F8 |
| --- | --- | --- | --- | --- |
| 1/500 | 1/250 | 1/125 | 1/60 | 1/30 |

Let's say, in our example above, you decide you want to use an even faster shutter speed of 1/500 sec. You now need to open your aperture up one more setting. But wait! Your camera may have a maximum aperture of F2.8, so you can't open it up more. If you go ahead and take the picture anyway, the result if an underexposed picture of your son frozen in motion.

Conversely, if you use a slower shutter speed, say 1/125 sec., but maintain the aperture of F2.8, now you have too much light and your picture is overexposed. For a correct exposure at 1/125 sec., you'll need to use an aperture of F4.

ISO

Where does ISO factor in all this? Remember, we used the speed of water flow as a metaphor for ISO. Using the smaller hose, if we could increase the speed of water flow, we would then be able to close the tap earlier and still end up with a full bucket! Similarly, increase the ISO, and you've increased the sensitivity of the image sensor -- i.e. it now needs less light to register an image. Suppose we meter a correct exposure at 1/30 sec., F8, at ISO 100. By increasing the ISO to 200, we can now use either a faster shutter speed at F8, or use a smaller aperture at 1/30 sec. and still obtain correct esposure. In our example above where we reached the limit of our camera's aperture range, we could have used a faster (higher) ISO to allow us to use 1/250 sec at F2.8.

Sounds great, isn't it? So, why don't we just use the highest ISO all the time? The answer is the dreaded 'noise.' The image sensor does not suddenly becomes more sensitive just because you tell it so by dialing in a higher ISO; it gains that increase in sensitivity at the cost of image quality.

Think of an image sensor as comprised of a matrix of photosites, with a photo sensor at each photosite capturing light for one pixel of information. Each photosite captures one pixel of info. When you dial in a higher ISO, you are increasing the sensitivity of the image sensor. Unfortunately the image sensor now captures not only more light signal but also more noise (any signal not generated by the light from your subject). It is the ratio of light signal to noise (S/N ratio) that determines the noise in your image. The more light signal (as opposed to noise) your image sensor is able to capture, the less noise in your image. Generally, the smaller the image sensor the worse the noise is. Most consumer digital cameras suffer from noise problems at high ISOs. Some digital SLR cameras have practically no noise at the higher ISOs; they, of course, use a rather large image sensor.

Common Complaints

Some of the most common complaints we hear from beginners are:

- Underexposure

- Overexposure

- Blurred Images

We already covered these above, but let's recap.

If your pictures are consistently coming out dark, then you are underexposing them. Remember, that setting your camera on Auto mode does not guarantee a correctly exposed picture if you ignore the warning the camera gives you when there's just not enough light. You need to increase the light source by using the on-camera flash, or an alternate light source, or move your subject to someplace brighter. Otherwise, an underexposed shot will result.

When there's too much light, and you cannot close the aperture enough and/or use a faster shutter speed, you end up with an overexposed image.

A blurred image results when you are either using a shutter speed that is too slow to avoid camera shake or too slow to 'freeze' motion. Use a faster shutter speed and adjust the aperture accordingly. A rule of thumb is to use the reciprocal of the 35mm equivalent focal length in use. Say you zoom to 125mm, then use a shutter speed of at least 1/125 sec. to avoid camera shake. To 'freeze' motion, a shutter speed faster than 1/60 sec. is usually necessary depending on the type and speed of the motion.

Exposure Bracketing

Sometimes, on one picture, you might have some areas of underexposure (usually in the shadows) and some areas of overexposure (we call them 'highlights'). If you meter for the shadows, the highlights will be 'blown' -- i.e. way overexposed. Concersely, if you meter for the highlights, the shadows will be way underexposed and you'll lose all details there.

One solution is to use exposure bracketing. Take one picture metering for the shadows; take a second picture metering for the highlights; take a third picture with an average metering. Load all three images into your favorite image editing software and overlay one on top of another. By carefully removing the portions of the image that are not properly exposed (or, conversely, by carefully combining the portions that are correctly exposed), you end up with one image with correct exposure throughout.

# What Is... Noise?

One of the major difference between a consumer digital camera and a digital Single Lens Reflex (dSLR) is that the former produces images with a lot of noise when using high ISOs and long exposure times, and the latter is practically noise-free. Noise is apparent by the presence of color speckles where there should be none. For example, instead of a blue sky, you notice faint pink, purple and other color speckles amongst the otherwise blue sky.

Just what is noise and how can we eliminate or minimize it? This article tries to explain all of this in as a non-technical way as possible. [This also means that the explanations may not be 100% technically correct. If you spot any mistake, please email me. Thanks.]

Digital Image

When we hear 'digital' we automatically tend to think of high quality. Digital sound does not degrade no matter how many times you play it. Digital images can be saved forever and will still print in its pristine form.

But the image in a digital camera is sourced from a non-digital component: the CCD or CMOS image sensor. Understanding how light striking the image sensor is converted into digital form makes clear what noise is and why it is present.

From Analog To Digital

An image sensor is typically comprised of a matrix of light sensors. A light sensor can be thought of as simply a device that converts light into an electric charge.

Each square of the image sensor matrix is a photosite, usually with one light sensor 'painted' on it. A photosite generally corresponds to one pixel in your digital image.

When light (photons) strike the image sensor, electrons are produced. These "photoelectrons" give rise to analog signals which are then converted into digital pixels by an Analog to Digital (A/D) Converter.

Causes of Noise

There are a number of sources of noise contamination.

Heat generated might free electrons from the image sensor itself, thus contaminating the "true" photoelectrons. These "thermal electrons" give rise to a form of noise called thermal noise or dark current.

Another type of noise is more akin to the 'grain' obtained by using a high ISO film. When we use a higher ISO, we are amplifying the signal we receive from the light photons. Unfortunately, as we amplify the signal, we also amplify the background electrical noise that is present in any electrical system.

In low light, there is not enough light for a proper exposure and the longer we allow the image sensor to collect the weak signal, the more background electrical noise it also collects. In this case the background electrical noise may be higher than the signal.

So why is using a larger image sensor better?

Each photosite itself generates electrical noise that can contaminate its neighbor. In a larger image sensor, the photosites can be physically further apart and thus be less affected by that contamination.

A larger image sensor also means that the photosite can be larger, thus have a larger light gathering capacity. It is therefore able to generate a larger signal to noise ratio.

That is why a digital camera with 6 million pixels crammed into a 1/1.8 in. image sensor has more noise (especially at high ISOs) than a 6MP digital camera using the much larger half-frame image sensor.

In-Camera Reduction Of Noise

Camera manufacturers have therefore incorporated in their firmware noise reduction algorithms that kick in when a slow shutter speed is used to try to reduce the noise. Depending on the quality of the algorithms, these work to a certain extent but do not completely remove the noise.

Noise Reduction Software

There is a number of image editing software that can be used to reduce noise in a digital image after you have taken them. Your image editing software may already have such an action, or you may download one free from the Internet. The better noise reduction software applications, such as NeatImage, take a long time to process one image and so may not really a viable solution if you have a lot of pictures with noise. They have their place though in a photographer's toolbox and for that one photograph that you have to take with noise or else miss an incredible shot, these software applications are your perfect noise reduction tools. In fact, no photographer should be without one.

Why Are dSLRs Practically Noise-Free?

If it has not yet occurred to you to ask it, you should. Really, why are dSLR images almost noise-free? The answer is simple: a larger image sensor!

See, with a larger image sensor, each pixel can be larger and each photosite can be a bit further away from its neighbor (of course, there is an optimum distance beyond which we'll have 'gaps'). This extra distance is often enough to prevent signal leakage from one photosite onto another -- hence much less to almost no noise!

Hurrah for Bigger Image Sensors!

Why therefore do camera manufacturers not use the bigger image sensors in consumer digital cameras? A bigger image sensor means the need for a bigger lens. Unlike film that can capture light incident on it at an angle, an image sensor requires that light falls on it straight on. Bigger lens add costs, need a bigger body, etc. etc. You get the idea. You quickly end up with a camera body the size of a... dSLR.

The biggest image sensor on a prosumer digital camera is 2/3 in. sized at 8.8 x 6.6 mm, though we are starting to see an APS size image sensor makes its way down.

The next size down is 1/1.8 in. (sized at 7.2 x 5.3 mm) and is prevalent in most of the 3MP, 4MP and 5MP consumer digital cameras today.

Notice how camera manufacturers have 'squeezed' more megapixels into the same 1/1.8 in. image sensor. That is one reason some people say that a digital camera at a lower megapixels resolution gives images that are more noise-free than one at a higher megapixels resolution -- on the same size image sensor. More megapixels on the same sized image sensor means the pixels are closer together -- thus more noise. Of course, better in-camera noise reduction algorithms in the newer digital cameras can counter this tendency toward more noise to a certain extent. Photographers must balance the advantage of higher megapixels versus more noise (albeit reduced with the in-camera noise reduction algorithms), although camera manufacturers leave us with few choices as they all move to the higher megapixels image sensor to compete with one another.

The dSLRs have image sensors that are much larger than 2/3 in. Some dSLRs have an APS-sized (or, 'half-frame', approx. 23.7 x 15.6 mm) image sensor.

When we talk about a 'full-frame' image sensor, it is in relation to a 35mm film and is therefore sized at 36 x 24 mm. Compare these with the 2/3 in. image sensors in prosumer digital cameras sized at 8.8 x 6.6 mm, and you'd agree that the size difference is indeed substantial. No wonder dSLRs produce practically noise-free images.

Are we ever going to see bigger image sensors in prosumer diigital cameras? Perhaps, because their camera body size may allow for it. But we may never see these larger image sensors in consumer digital cameras unless new bigger lenses are designed cheap enough. Don't hold your breath, though. It may be more realistic for now to wait for better noise reduction algorithms that may make the noise level acceptable.

What Can You Do?

There are a number of things to remember about noise:

- A slow or long exposure introduces noise.

- A higher ISO introduces noise.

- Noise Reduction Software will clean up the noise in some images, and sometimes it's done so well that you can't tell the original image had unacceptable noise level in it.

- If you must have the option of using high ISOs (e.g. to capture fast action in low-light situations), then get a dSLR. Since many amateur photographers purchase 35mm SLR cameras today (especially since they are relatively cheap), I don't see why they should not be able to use a dSLR as easily. The only roadblock to owning a dSLR for many today is its high price, though even that is steadily coming down as competition heats up among camera manufacturers. A dSLR currently costs about $1000+, body only.

Noise is a fact of life in consumer and prosumer digital cameras, and is going to stay with us for some time longer until camera manufacturers engineer better and small noise-free image sensors. Until then, what can you do to reduce the amount of noise in your digital images?

- Avoid slow or long exposures.

- Avoid using high ISOs.

- When comparing digital cameras, all things being equal, favor the one with the larger image sensor. For example, Camera A may be 3MP using a 1/2.7 in. image sensor (sized at 5.3 x 4.0 mm), while Camera B may also be 3MP but uses a larger 1/1.8 in. image sensor. Camera B will probably produce images that have less noise than Camera A.

- Purchase a noise reduction software. This will allow you to take pictures and not miss some great shots. You'll find that a good noise reduction software usually does a pretty good job of 'cleaning' up the noise to an acceptable level in most of your noisy images.

Here is a comparative illustration of the approximate sizes of the currently most popular image sensors:

| Image Sensor | Size (approx.) |
| --- | --- |
| full frame | 36 x 24 mm |
| half frame | 24 x 15 mm |
| 2/3 in. | 8.8 x 6.6 mm |
| 1/1.8 in. | 7.2 x 5.3 mm |
| 1/2.7 in. | 5.3 x 4.0 mm |

# Fireworks

A good fireworks picture can be one of the most spectacular picture you take, and the opportunity comes only once or twice a year. How can you take good fireworks pictures with your digital camera? It's not too difficult and you don't need an expensive digital camera to do so -- but there are a few pointers to keep in mind.

Fujifilm FinePix F10
Focal Length = 20.1mm, Shooting Mode = Night Scene (Standard) 
Shutter Speed = 3 sec., Aperture = F4.7, ISO = 80
Sharpened in Photoshop Elements

Shutter Speed

First, your camera must be able to keep the shutter open for a few seconds of exposure since you want to capture both the initial explosion as well as the luminous trails created by the falling burning particles.

If your camera has a Bulb function, then you are in luck. Switch to Manual, set the shutter to Bulb, and the shutter will remain open as long as you press the shutter release button down, and close when you release it. Keep the shutter open and record a burst from the moment it explodes to the time when it starts to fade. If you are taking only one burst, then you're done. If you want to take multiple bursts on the same frame, just put the lens cap (or simply a thick cardboard or even your hand) on instead of releasing the shutter release button. Take the cap off for the next burst and continue this cap on/off process until you are satisfied.

Depending on how dark the conditions are, you might need to experiment on how long to leave the shutter open. Since you would want to include foreground buildings to situate the fireworks, leaving the shutter open for too long may cause these buildings to be overexposed.

Fujifilm FinePix F10
Focal Length = 20.1mm, Shooting Mode = Night Scene (Standard) 
Shutter Speed = 3 sec., Aperture = F4.7, ISO = 80
Sharpened in Photoshop Elements

Aperture

You might want to experiment with the aperture to use. A small aperture such as F8 will make the burst trails thin and dark, whereas a larger aperture such as F2.8 will make them thicker and lighter. If your digital camera does not have very slow shutter speeds, or shoots only in Program AE mode, it is a bit more challenging. The aperture used would most probably be F2.8 resulting in thick and light lines, which are less appealing. If you can, zoom in for this usually also closes down the aperture.

Fujifilm FinePix F10
Focal Length = 20.1mm, Shooting Mode = Night Scene (Standard) 
Shutter Speed = 3 sec., Aperture = F4.7, ISO = 80
Sharpened in Photoshop Elements

Exposure Mode

If you do not have Bulb setting, then switch to Manual, or Shutter-Speed, or Aperture-Priority mode. Most Point-and-Shoot digital cameras will not have any of those modes, but will usually have Night Scene mode, or Slow Shutter scene mode. Choose whatever mode that will give you a shutter speed of about 3 sec. and more.

In the image examples in this tutorial, I set the camera (Fujifilm FinePix F10) to Night Scene mode which gave me a shutter speed of 3 sec. I zoomed in a bit and that restricted the aperture to F4.7. I also set the ISO to 80 (the lowest on that particular camera) to prevent overexposure when using long shutter speeds.

The camera was set on a tripod with the center pole extended above the balcony railings of the room I was staying in at the Disney Contemporary Resort. The room was on the 7th floor of the tower and faced the Magic Kingdom [you gotta ask for it or your room will face the lake or garden]. Because of the angle, I could not include the Cinderella Castle in the shots, so included the Space Mountain instead.

Fujifilm FinePix F10
Focal Length = 20.1mm, Shooting Mode = Night Scene (Standard) 
Shutter Speed = 3 sec., Aperture = F4.7, ISO = 80
Fireworks from 3 different shots sharpened and combined in Photoshop Elements

Tips

- Because you'll be using very slow shutter speeds, you'll find that camera shake is a major problem. Handholding your camera will almost guarantee trails that look shaky. So, use a tripod. Don't forget that with most tripods you can also extend the middle pole to allow your camera to reach over a balcony railing or other obstacle.

- Likewise, if your camera allows it, use a release cable or remote controller to reduce camera shake. It's tricky to use a self-timer and capture the bursts just when you want to.

- If your camera has a long zoom, then you can fill in the whole area with a single burst. If your camera does not zoom in much, then consider taking two or more bursts on the same frame (if your camera allows multiple exposure on the same frame).

- Don't trip the shutter release as soon as the fireworks explode because the extreme brightness of the initial explosion will generally result in overexposure. Instead wait for the trails to bloom fully and then open the shutter to catch the bloom and the falling trails.

- You may also want to wait for the last explosions in a series, for too many explosions may also result in overexposure.

- No double exposure function or Bulb on your digital camera? Don't worry, the magic of digital photography allows you to sandwich a number of bursts afterwards in an image editing software, such as Photoshop. You can combine bursts as creatively as you desire, even making some appear smaller or larger to achieve a balanced and pleasing composition.

- As you can see, a camera that allows you to use long shutter speeds, adjust aperture, has a wide-angle lens (especially if you are going to be relatively close to the launching point) and has a remote controller is ideal. But if your camera does not have all those features, as mine did not, don't let that stop you -- you can still take spectacular fireworks shots with a little experimentation!

# Fill-In Flash

If you have pictures where the foreground is too dark while the background seems to be correctly exposed, then using fill-in flash may be a technique worth learning about. Using fill-in flash, as the name implies, is to use flash to fill-in light where it is deficient. This is usually the situation when you are in the shadows and shooting through objects in the foreground that are acting as a frame.

In the photo above, I was standing in a shaded spot and using the foliage as a frame. Without fill-in flash to illuminate the foreground, the latter would be quite dark. Shutter speed: 1/180 sec., Aperture: F2.8, ISO 50 with fill-in flash. (Exposure compensation was -0.7EV but that was because I forgot to reset it for a previous picture.)

Instead of the leaves, imagine a person stood in the foreground. Unless you wanted to silhouette the person (this can be quite effective in producing a dramatic picture), fill-in flash would throw in just enough light to illuminate the face.

A most pleasant effect can sometimes be achieved by photographing a person against the sun (or another strong light source). The light behind his or her head creates a halo around the edge of the hair. The face would be quite underexposed but for fill-in flash. Shooting against the sun also avoids your subject squinting in the sun.

To use fill-in flash, set your digital camera so that it is forced to fire the flash. Do not confuse with Night Shot scene mode which is similar but generally used at nightime for night portraits. In this case the flash fires to illuminate the foreground, while the shutter remains open a while longer to expose the background. You need to keep the camera steady until after the flash has fired and the shutter has closed.

# Frame Your Picture

The use of a frame can turn an otherwise plain picture into a pleasing one. Usually a foreground element is used to create the frame. Examples are an overhanging tree branch, a window frame, a door, arches, a fence, rows of trees, etc.

A frame serves to isolate your main subject and create an interesting composition. Oftentimes, it's the feeling of depth you're after. You may choose to keep the detail of the frame (as in a wall, fence or branch), or let the frame go completely dark (as in a window frame).

When you are composing a shot, keep your eyes open to foreground (and sometimes, even middle or background) objects that can be used as a natural frame. You may need to physically back up a bit, kneel down for a more pleasant composition, or zoom in to include your subject within your chosen frame.

Bridge at Kariya Park:
Here, the tree and its branches serve as a natural frame to the bridge.
The tall grass and rocks contribute to the frame effect and give the pleasing impression that we are 'peeking' into a peaceful scene.
Canon PowerShot S50
Program AE, Evaluative, Auto WB
7.1mm, 1/250 sec., F2.8, ISO 50

By cropping tightly, a frame is created and draws attention to the bell. This picture would have been more effective if the background was blurred (by using a larger aperture combined with a long focal length).
Nikon Coolpix 8700
13.7mm, Programmed Auto, Partial, 1/95.7 sec., F3.5 and ISO 50
Levels adjusted in Photoshop Elements

A Word Of Caution About Artificial Frames

Framing can also be done after the fact by combining two or more pictures in an image editing software. Very dramatic effects can be accomplished this way and you are only limited by your imagination.

However, a word of caution is necessary here. Though there is nothing wrong with combining different images to create another one, be very careful about employing artificial frames on recognizable scenes and landscapes.

As an extreme example, don't frame the Statue of Liberty by the St. Louis Arch. Unless your purpose is to create dissonance within your viewers' minds, that picture will just look strange.

So, before you take that shot next time, look around and see if there is an object that you can use as a natural frame. Take a picture without the frame, and another one with the frame, and compare. Betcha, you'll like the one with the frame better!

# Sharpen Your Photos For Display

Most of us upload our pictures or print them the way they were captured by our digital cameras. We read and hear of a post-processing technique called "Sharpening" using "USM" (or "UnSharp Mask") and decide it is just too difficult for us. If you are comfortable using a computer and can load an image into an image editing software, such as Photoshop Elements, then be prepared for a pleasant surprise!

Most cameras -- especially in the "pro" category -- will capture an image without applying any sharpening to it, resulting in an image that appears "soft" or even slightly "out of focus." Others will apply a certain degree of sharpening to the images and output crisp looking images. Most beginners prefer the latter type, while most advanced photographers prefer the former. Here's why.

When a camera processes your images, it is in effect deciding the amount of sharpening to give your images without your input. Some cameras do a good job at it, others do too much of it. It is akin to processing labs a couple of years ago that treated all your negatives the same way, and all got printed with the same adjustments. If you go back and look at your old prints, chances are they all have a bluish tint to them. As competition heated up, processing labs started to examine each print individually and making adjustments and corrections to each, resulting in much better color rendition. This is akin to you sharpening your own images the way you like them.

Of course, if you have to do that for every single picture you take, it is really not worth it. Truth is, most pictures don't need it. It is only the few pictures you really want to highlight, either on screen or in print, that will benefit most from sharpening (and other adjustments).

If your camera produces "soft" images (because it does not sharpen for you), then you are in fact in luck. Using an image editing software such as Photoshop Elements, you can apply sharpening yourself to selected pictures for maximum impact, either for screen display or for printing.

In fact, if you are in the habit of printing unretouched pictures from the different digital cameras and then comparing the prints to see which camera is better, you are making a grave mistake. Unless you have sharpened the images individually first, the comparison is really not a valid one, and you'll end up with wrong conclusions.

Sharpening for screen display and sharpening for prints are also two different things. So, let's understand what sharpening is and how to do it for screen display. (I cover for screen display here, and for prints at a later date.)

What Is Sharpening?

Sharpening is enhancing the edges of an image. This edge enhancement technique is called unsharp masking and involves isolating the edges in an image, amplifying them, and then adding them back into the image. In Photoshop, amplifying the edges is accomplished by locating pixels of greatest contrast, lightening one side and darkening the other. Once sharpening is appropriately applied, people's reactions is usually, "Wow! It jumps right out at you."

Perhaps the simplest way to understand sharpening is to attempt one in Photoshop Elements.

Here is our original unsharpened image we will be working with in this tutorial. It is a "soft" image, even appearing out of focus in some places. Don't be deceived by appearances.

Original unsharpened image
Canon PowerShot S50
Program AE, Center-weighted averaging, Auto WB
7.1mm, 1/60 sec., F2.8, ISO 50, Flash ON

Make A Copy

Open Photoshop Elements and do File - Open - select the picture you want to work with.

Immediately, save it as a different name, with a psd (Photoshop) extension. For example, if your original image was named IMG0001.JPG, then save it as IMG0001.psd (i.e. use the default extension of the image editing software you are using). This way, you will preserve the original file (IMG0001.JPG).

If the Layers Panel is not on your desktop, drag it from the tabs at the top into your work area. If necessary, drag the bottom edge down so you can see more than one layer. You will see your picture (probably named "Background") in a layer.

Double click on that layer and rename it as original. (You could leave it as "Background" but renaming it is a good habit, since you may want to add a real background in some cases.)

Now, drag it to the new layer icon (the middle one at the bottom of the Layers Panel) to create a new layer, original copy.

Click on the original copy layer to select it as the one you want to work with. (Remember, we never want to muck with the original layer.)

Your Layers Panel should end up looking like this:

Sharpen

If not already selected, click the original copy layer to select it.

From the menu bar, select: Filter - Sharpen - Unsharp Mask... and type in Amount = 200%, Radius = 1.2, Threshold = 4. Click on Preview to view the original and sharpened version. Yeah!, I hear you say already. You may select different values, though the above values are a good starting point.

Separate Darken and Lighten Pixels

Now, copy the sharpened original copy layer by dragging it to the new layer icon. You'll end up with original copy 2.

Double click on original copy and rename it darken.

Double click on original copy 2 and rename it lighten.

Select the darken layer by clicking on that layer.

Set the blending mode to darken: See where it says, Normal in a drop down box at the top of the Layers Panel? Click on the down arrow and select "Darken":

Select the lighten layer by clicking on that layer.

Set the blending mode to lighten: Click on the down arrow and select "Lighten":

What we have done is separate the darken and lighten pixels into two layers. Now, we can control each layer separately.

Adjust Sharpness

If you are following this tutorial in your own Photoshop Elements and using our image, you'll notice that the lighten pixels (the highlights) are a bit too bright. (You might have to zoom in 200% to see things clearer.)

Click on the lighten layer to select it and adjust the Opacity until you are satisfied. (Ensure Preview is ON.) In my case, I select 26%.

Likewise, select the darken layer and adjust the Opacity until the darken pixels (the dark edges) just "jump out." I select 90%.

Each picture you sharpen will require different values, so if you are using your own image, adjust to your own liking.

Here is my final result:

Sharpened image

And, here is the original version again for comparison:

Original unsharpened image

There, you have it. Pretty simple, huh?

Save this final work (in our example, as IMG0001.psd).

Render unto Caesar what belongs to Caesar: I followed the sharpening technique spelled out in Russell Brown Tips & Techniques. Russell Brown explains the same sharpening technique and other cool Photoshop tips in easy-to-follow QuickTime videos.

# Area Focus

Most of the newer digital cameras include three focusing modes: Center Focus, Area Focus and Multi Focus.

Center Focus will focus on the center of the frame.

Multi Focus (aka AiAF and other acronyms) automatically selects between a number of AF frames and finds the most contrasty subject in or close to the center of the frame to focus on. If your main subject stands beside some other object (behind or in front of your main subject) of greater contrast, your main subject will be out of focus. (If you use Multi Focus as your default focus mode, this may explain why some of your shots are out-of-focus.)

Area Focus deserves some explanation. I believe it is a focus mode that few uses, but there are certain situations where Area Focus is perfectly suited for and can help you capture the shot.

Center Focus
If you look in the LCD of your digital camera, there will be a rectangle at the center of the screen, commonly called the AF frame. This AF frame depicts the zone where the camera will focus on.

If your subject is smack in the middle of the screen, well and good.

But, what if your subject is not in the center of the screen?

Well, most of the time, we use a handy technique called Pre-Focus. By pre-focusing, I mean that we swivel the camera slightly so that the center rectangle is on the subject, depress the shutter release half-way to lock focus, then swivel the camera back for our desired composition, and depress the shutter release fully to take the picture.

Pre-focusing is used extensively in action photography when you know that you want to take the picture at a certain fixed spot. In this case, you can pre-focus on that spot by a half-press of the shutter release, then follow the action and depress the shutter fully when your subject reaches the fixed spot. Examples of a fixed spot can be a basketball hoop, a finish line, a spot right in front of you, etc.

Area Focus
Area Focus extends Center Focus by allowing the AF frame to be moved anywhere on the screen. So now, if your subject is to the left of center, instead of moving the camera left, pre-focusing and then moving the camera back, all you do is move the AF frame left instead while keeping the camera still!

On some cameras, you can move the AF frame easily without taking your eye off the LCD or viewfinder. On others, you need to go into the menu, set Area Focus mode and position the AF frame where you want it to be using the four-way controller. And repeat that procedure for every picture where you want to move the AF frame. No wonder we don't use this focus mode as often, preferring instead to use pre-focusing.

Even then, there is at least one photo situation where Area Focus is perfect for: macro photography. When taking a close-up you may want to compose your picture so that the subject is at one corner of the frame. Since precise focus is paramount in close-up shots (depth of field is very shallow), pre-focusing correctly can be difficult to achieve since when you recompose you may end up moving the camera a little closer or further to your subject. The result is an out-of-focus picture (your main subject will be out of focus though other parts of the picture may be in focus).

This is where area focus comes in and saves the day.

By moving the AF frame on your subject to the corner of the frame so that it covers your subject, you can maintain your composition without having to move your camera, lock your tripod, set your self-timer, and take the shot -- for a spot on razor sharp image.

An Example
Here is an example of how Area Focus helped me achieve sharp focus in a macro shot. The AF frame in the center of the frame encompasses both the fir needles and the leaves. Where will the camera focus on: the fir needles (which is what I want) or the leaves in the background? In the first picture, using Center Focus, the fir needles do not come out as sharp as I want; instead the leaves get the focus.When I try to pre-focus, moving the camera back to my composition is enough to lose precise focus.

Fujifilm FinePix S5000
Programmed Auto, Multi-Pattern metering, Macro ON
19mm, 1/180 sec., F3.2, ISO 200

I could switch to Manual Focus but I don't trust my eyes (even with a central enlarged image). I could switch to Multi Focus and let the camera decide on the most contrasty area. But why leave this to chance? I decide to use Area Focus instead. In the second picture, I move the AF frame to the bottom left, completely covering the fir needles. Result: fir needles in focus, leaves in background nicely blurred.

Fujifilm FinePix S5000
Programmed Auto, Multi-Pattern metering, Macro ON
19mm, 1/170 sec., F3.2, ISO 200

So, why not try Area Focus, if your camera allows it? You may find that, besides macro photography, there are other instances when it is the appropriate focus mode to use.

# Center AF

Most digital cameras use contrast detection to auto focus (AF). Usually, the focus point is a small rectangle in the middle of the viewfinder frame (Center AF), though many digital cameras now also offer additional AF points (Multi-Point AF).

Center AF

If you look in the LCD monitor of your digital camera, there will usually be a rectangle at the center of the screen, commonly called the AF Frame. When your digital camera is set to Center AF mode, this rectangle in the middle of the screen is your AF point.

Multi-Point AF

Multi-Point AF automatically selects between a number of AF points (the most common seems to be 5 or 9 AF points -- i.e. 4 or 8 AF points clustered around a center focus point) and finds the most contrasty subject among those AF points.

An Example

If your default AF mode is Multi-Point AF and some of your shots are sometimes out of focus, it may be that Multi-Point AF is the culprit.

Here is an example of how Multi-Point AF focused on the more contrasty background well behind my main subject (the lone green leaf left on the bush).

Even though the green leaf is in the center of the frame, Multi-Point AF has chosen to focus on the more contrasty background around it instead.

The result is that the ground of fallen leaves come out in focus, while my main subject is out of focus.

Sony DSC-P150 Cyber-shot: 5 Area Multi-Point AF
Program AE, Multi-Pattern Metering, Macro ON
7.9mm, 1/250 sec., F2.8, ISO 400

I took a couple more shots with the same result before I realized what the problem was. I immediately went into the camera's menu to switch AF mode from Multi-Point AF to Center AF. This time, I got the result that I was after: the lone green leaf in focus against a nicely blurred backdrop!

Sony DSC-P150 Cyber-shot: Center AF
Program AE, Multi-Pattern Metering, Macro ON
7.9mm, 1/200 sec., F2.8, ISO 400

Summary

Multi-Point AF works pretty well usually but sometimes the camera will focus on something else besides your main subject if that something else happens to fall under one of the AF points and is also more contrasty than your main subject. In these instances, switch AF mode to Center AF, point the AF Frame on your subject and half-press the shutter release button to lock focus on your main subject, then reframe, if necessary, before fully pressing the shutter release button to take the shot.

Some more advanced digital cameras also allow you to move this AF Frame away from the center to other parts of the screen (so your subject does not need to be centered). Want to learn more? Read our tutorial on Area Focus.

# Levels

Digital Photography is more than just using a digital camera; it also entails some post processing in an image editing software to the image you captured with your digital camera. And it is often not as complicated as you might think it is -- or as some make it to be. In this tutorial, we look at "Levels" and cover the basics of what you need to know about adjusting levels in Photoshop Elements. It's a simple 3-step process that takes barely a minute to perform but that can help you bring back to life pictures you thought forever ruined.

Adjusting Levels

You've read the notes that accompany some pictures: "Some adjustments to levels peformed" and have probably been mystified as to what exactly that entailed.

Adjusting levels is a 3-step process:

- Adjust for the shadows;

- Adjust for the highlights; and,

- Adjust for the midtones.

Let's look at a picture that seems "ruined," in this case, it seems under exposed, i.e. too dark:

If, in the past, you have just thrown up your hands in the air and bemoaned how bad your digital camera was, perhaps you were a bit too hasty. We will recover this picture using our 3-step levels adjustment process.

[If you want to follow along with this tutorial, download the above picture now and open it in Photoshop Elements.]

In Photoshop Elements, select Enhance - Brightness/Contrast - Levels... A histogram pops up on your screen and looks like this:

Don't panic! The histogram is simply a representation of all the pixels in the picture. A quick glance at it tells us that most of our pixels are toward the shadows (to the left). Along the horizontal axis, there are 3 triangular markers (really, sliders): use the black one at the left to indicate where the darkest group of pixels start in your picture; use the white one at the right to indicate where the brightest group of pixels start in your picture; and use the shaded one in the middle to indicate the midtones. It's that simple!

Let's apply our 3 steps:

1. Adjust for the shadows. We want to move the black marker to the first group of dark pixels. In this case, since the black marker is already pointing to the first group of dark pixels, we will leave it where it is, i.e. at the 0 mark.

2. Adjust for the highlights. We want to move the white marker to the first group of bright pixels. In this case, we move the white marker to the left to a point where we feel the first bright group of pixels start, around the 172 mark:

By doing this, notice how the picture has changed already for the better, with "true" highlights:

3. Adjust for the midtones. Click on the shaded marker and drag it to the left or right until you are satisfied with the result, in our case to somewhere around 1.98 (adjust to your own personal preference):

Click OK and save your "restored" picture. Nothing wrong with this picture now -- expect for the clutter on my desk, that is!

There, that was easy, eh? Get into the habit of bringing your picture into levels and adjusting it for shadows, highlights, and midtones before you apply sharpening to it (another easy process):

# Histogram

What The Histogram Is

The histogram is simply a graph that allows you to judge the brightness of an image. You can think of the area under the graph as comprising all the pixels in your captured digital image. The left side of the histogram depicts how many "dark" pixels you have captured; the right side, how many "bright" pixels you have captured.

Let's look at the histograms of three pictures I took to illustrate how to read and use a histogram:

| Too Dark | Just Right | Too Bright |
| --- | --- | --- |

The left histogram indicates you have most of your pixels toward the dark. The pixels also touch the left edge, indicating underexposure [note 1]. If Goldilocks had stumbled onto this histogram while taking a picture, you can hear her saying something to the effect: "This picture is too dark."

The right histogram indicates you have most of your pixels toward the highlight. The pixels also touch the right edge, indicating overexposure. Looking at this histogram, Goldilocks would wail something to the effect: "This picture is too bright."

The middle histogram depicts a correctly exposed picture with the pixels mostly in the middle, i.e. neither too dark nor too bright. (Goldilocks: "This picture is just right.") Notice a few pixels touch the dark edge, so we should also expect a few very dark spots in the picture.

Perhaps the best way to understand the histogram is to look at the sample shots that go with the above three histograms.

Too Dark

Shutter Speed 1/180 sec., Aperture F2.8, ISO 50

Aim your camera out the window toward a street light at night or at a scene mostly in deep shadows, like the one above. Look at the histogram produced:

At a glance, we can see that most of the pixels tend toward the dark (D) side. The pixels also touch the left edge, so we know that there are underexposed ("too dark" -- thank you, Goldilocks) areas in our picture. To obtain a correct exposure, we can do a couple of things, taking the above picture as example:

- In Program AE mode, dial in a positive exposure compensation in +1/3EV steps. (For the above picture, we'll dial in +1/3EV.)

- In Manual mode, dial in a slower shutter speed, while retaining the same aperture. (For the above picture, the shutter speed used was 1/180 sec., so we'll dial in a slower shutter speed at 1/125 sec.)

- In Manual mode, dial in a wider aperture, while retaining the same shutter speed. (For the above picture, an aperture of F2.8 was used, so we'll set a wider aperture of F2.0.)

Depending on your camera, you may or may not be able to do one or more of the above. For example, most consumer digital cameras have a maximum aperture of F2.8, so we would not be able to try the third option.

What we are doing is telling the camera to ignore its light meter and allow the light to register on the image sensor for a while longer. We want the histogram to shift right. Check the new histogram produced and adjust again if the histogram indicates the picture is still too dark.

Too Bright

Shutter Speed 1/56 sec. Aperture F3.2, +1EV, ISO 200

On a bright sunny day, aim your camera at a landscape photo making sure to include much of the bright sky or areas bathed in bright lights or sunlight, as in the picture above. Here, I am exaggerating with a way overexposed shot (I used +1EV to force an overexposure) to illustrate the point. Look at the histogram produced:

At a glance, we can see that most of the pixels tend toward the highlight (H) side. The pixels also touch the right edge, so we know that there are overexposed ("too bright" -- thank you again, Goldilocks) areas in our picture. To obtain a correct exposure, we can do a couple of things, taking the above picture as example:

- In Program AE mode, dial in a negative exposure compensation in -1/3EV steps. (For the above picture, we'll dial in -1EV since I gave it a +1EV to force overexposure.)

- In Manual mode, dial in a faster shutter speed, while retaining the same aperture. (For the above picture, the shutter speed was 1/56 sec., so we'll dial in a faster shutter speed at 1/125 sec.)

- In Manual mode, dial in a narrower aperture, while retaining the same shutter speed. (For the above picture, the aperture used was F3.2, so we'll set a narrower aperture of F5.6.)

Depending on your camera, you may or may not be able to do one or more of the above. Since the exposure settings chosen by the camera for the above picture are well within the limits of our digital camera, we have quite a bit of leeway here to play with the exposure settings.

What we are doing is telling the camera to ignore its light meter and allow the light to register on the image sensor for a shorter time. We want the histogram to shift left. Check the new histogram produced and adjust again if the histogram indicates the picture is still too bright.

Balanced Exposure

Shutter Speed 1/60 sec., Aperture F2.8, ISO 50 with Flash

This is an indoors shot using flash. Without flash, the picture would have been underexposed (i.e., "too dark" -- thanks, G). The areas reached by the flash are exposed properly, but notice that the areas too far away for the flash to reach are dark. Look at the histogram produced:

At a glance, we can tell that the picture has neither too dark nor too bright pixels (i.e. it is "just right" -- Thanks again, G.) with most of the pixels in the middle of the graph. There are a few pixels touching the dark edge, just as we expected.

You might want to try dialing in a positive exposure compensation in this case to see if the histogram improves. For the above picture, we'll dial in +1/3EV and see if this shifts the histogram to the right slightly. Just be careful not to overexpose it!

Breaking The Rules

Shutter Speed 1/160 sec. Aperture F8, +1EV, ISO 200

If you are aiming your camera at a landscape with dark shadows and very bright sky, you may have both dark shadows and blown highlights (histogram shows pixels touching both left and right sides). In this case, you may have to decide which is more important to you. You are in control. In the picture of the ferris wheel above, I even dialed in a +1EV exposure compensation to try to bring out some details in the shadows, but this simply caused the bright sky to become washed out. It would have been better to expose for the sky and allow the girders to go black completely.

Shutter Speed 1/100 sec., F2.8, ISO Auto

For example, when we are taking a sunset, we usually do not really care about preserving details in the shadows. In fact, we do want the foreground to go completely dark. In this case, we expose for the bright area (the setting sun) and let the shadows go dark for a dramatic sunset picture. In fact, we may want to purposefully break the rules and take a couple more of that sunset by dialing in negative exposure compensation values. This is because sunsets usually look better (with deeper colours) with a slight underexposure. (A note of caution about sunsets: never look at the sun directly, whether with the naked eyes or through the lens of a camera. The image sensor of a digital camera may also get damaged by the direct rays of the sun. When is it safe to point the camera at the sun for a sunset picture? When your eyes can look at the setting sun without requiring you to squint or block your eyes.)

If, on the other hand, you are more interested in preserving the details in the shadows, you would expose for the shadows and let the highlights be blown. Again, you maintain control, not abdicating it to the histogram.

Picture Blending

An advanced technique you can use is to take two pictures, one exposed for the shadows and a second one exposed for the highlights (the use of a tripod is essential here). Then you would edit, erase the under and overexposed portions of each picture, and carefully merge the two pictures in an image editing software (such as Photoshop) to obtain a picture that is properly exposed in both shadows and highlights!

Of course, this technique may be extended to taking as many pictures as you feel necessary depending on how many difficult areas you have to deal with. You certainly need to be pretty good at Photoshop.

Be careful, though, because this blending of many pictures sometimes result in pictures that the brain innately recognizes as being "unnatural."

Summary

The histogram is a handy graph that alerts you to the two extreme cases of underexposure and overexposure in your pictures.

In severely underexposed pictures, you lose details in the shadows. In severely overexposed pictures that result in blown highlights, you lose details in the highlights.

If the image is too dark (histogram touches the left side), dial in a positive exposure compensation value in +1/3EV steps, and see if the histogram shifts to the right. Some pictures might require up to a +1EV exposure compensation to preserve details in the shadows.

If the image is too bright (histogram touches the right side), dial in a negative exposure compensation value in -1/3EV steps, and see if the histogram shifts to the left. Some pictures might require up to a -1EV exposure compensation to preserve details in the highlights.

Most digital cameras allow you to dial in an exposure compensation between -2EV to +2EV, in 1/3EV steps.

We have also seen that there is no good or bad histograms, per se. A histogram simply tells us how our picture is exposed and we retain the control to decide whether -- and how -- to adjust the exposure. We might decide to compensate for the under or overexposure, or we might even force an underexposure if that is the effect we are after.

# Take TV Pictures

Taking pictures from the television screen using your digital camera is quite easy.

If you have ever tried taking pictures straight from your television screen, you might have sometimes noticed horizontal lines running up and down the picture. That is because a TV screen is "painted" one pixel at a time from top to bottom. The lines appear on your pictures when you use a shutter speed that is too fast and that "freezes" the lines on the television image.

Kodak Easyshare DX6490
Shutter-Priority mode, Spot
9.8mm, 1/90 sec., F3.2, ISO 80

In this One-Pager™ tutorial, we'll give you the one simple trick that you need to know to be able to take television picture using your digital camera. Ready? All right, simply use a shutter speed of 1/30 sec. and preferably slower. The idea is to use a shutter speed that synchronizes with the the painting of the TV image and voila -- no moving horizontal lines.

You might want to experiment with different shutter speeds, such as 1/15 sec., 1/25 sec., (but less than 1/30 sec.) to see where you get optimum quality on your TV using your digital camera. Of course, to be able to set the shutter speed on your camera, you need to be able to switch to Shutter-Priority mode. The following pictures were taken with the Kodak Easyshare DX6490, and I find that a shutter speed of 1/15 sec. is optimum. With 1/30 sec., a few lines is apparent at larger image size.

For images of a computer screen, try 1/20 sec. and slower, and get a meter reading directly from the screen.

Dora
Shutter-Priority, Spot
9.8mm, 1/15 sec., F3.6, ISO 80

Max & Ruby
Shutter-Priority, Spot
9.8mm, 1/15 sec., F5.6, ISO 80

Smallville
Shutter-Priority mode, Spot
11.5mm, 1/30 sec., F3.2, ISO 80

Star Trek Enterprise
Shutter-Priority mode, Spot
11.5mm, 1/30 sec., F3.2, ISO 80

# What Is... ISO

|  | ISO Sensitivity | Auto; ISO 100, 200, 400, 800 equivalent |  |
| --- | --- | --- | --- |

ISO sensitivity expresses the speed of photographic negative materials (formerly expressed as ASA).

Since digital cameras do not use film but use image sensors instead, the ISO equivalent is usually given.

What ISO denotes is how sensitive the image sensor is to the amount of light present. The higher the ISO, the more sensitive the image sensor and therefore the possibility to take pictures in low-light situations.

And, where you would have needed to physically change to a different roll of film if you wanted a different ISO speed, digital technology allows you to simply dial one in. In this way, you can record images taken at different ISO speeds on the same memory card.

ISO Speed & Exposure

ISO speed affects the shutter speed / aperture combinations you can use to obtain correct exposure.

Suppose your digital camera's light meter warns you there is not enough light to correctly expose a scene. You could use the on-board flash, but let's suppose again it's not allowed (like in a concert or indoors recital).

You would then need to use a higher ISO. Set on "ISO Auto" mode, your digital camera will automatically select a higher ISO. Otherwise, you can manually select the next higher ISO and see if the increased sensitivity allows you to obtain a correctly exposed picture. If it does, you can now take a correctly exposed picture.

Similarly, if you find the camera is using a shutter speed that is too slow (1/60 sec. and slower) to handhold the camera steady and shake-free (thus resulting in blurred pictures), and you cannot open up the aperture anymore, and you do not have a tripod or other means to hold the camera steady, and you want to capture the action, etc. etc. -- then you might select the next higher ISO which will then allow you to select a faster shutter speed.

ISO Speed & Noise

However, all this increase in sensitivity does not come free. There is a price to pay with your image appearing more noisy.

See, when you boost the sensitivity of your image sensor by selecting a higher ISO, the image sensor is now able to record a fainter light signal. However, it is also true now that it will record fainter noise, where noise is any signal that is not attributed to the light from your subject. Remember that an image sensor is still an analog device and it generates its own noise, too! The increased sensitivity allows the image sensor to record more light signal and more noise. The ratio of light signal to noise (S/N ratio) determines the "noise" in your resultant image.

An image sensor is usually calibrated so that it gives the best image quality (greatest S/N ratio) at its lowest possible ISO speed. For most consumer digital cameras, this value will be expressed as ISO 50, ISO 64 or ISO 100. A few digital cameras use ISO 200 as their lowest ISO speed.

Just as with its film counterpart, an image sensor will exhibit "noise" (comparable to "graininess" in film) at the higher ISO speeds. Unlike film, where graininess can sometimes contribute to the mood of the image, noise produced by an image sensor is undesirable and appears as a motley of distracting coloured dots on your image.

ISO Speed & Image Sensor Size

The size of the image sensor determines the ISO speed range that a digital camera can use without suffering from undue noise. One reason for this is because the pixels on the larger image sensor can be larger and therefore receive more light, and thus have a greater signal-to-noise (S/N) ratio (for more information on noise, see our tutorial: What Is... Noise?).

If we take two image sensors, each with 4 megapixels resolution, but of different sizes, the 4 megapixels image sensor that is smaller will exhibit more noise at higher ISOs than the larger one.

| 1/1.8 in. (7.2x5.3mm) | APS-sized (23x15mm) |
| --- | --- |
| 4 million tiny pixels crammed into a 1/1.8 in. image sensor cannot compete in image quality with 4 million large pixels on an APS-sized image sensor. |  |

Most consumer digital cameras use 1/1.8 in. (and smaller) image sensors, so noise at high ISO is a problem. Though they will allow you to use a high ISO (such as ISO 400), the resultant image will usually be very noisy.

A 100% crop of a picture taken at night at ISO 400 (1/1.8 in. image sensor) Click on image for original size [warning: large file]

A digital SLR (dSLR), on the other hand, uses a large image sensor, usually full frame (24x36 mm) or APS-sized (half-frame). Noise is rarely a problem and the use of a high ISO 400 results in images with barely noticeable noise.

| ISO 400 | ISO 800 | ISO 1600 |
| --- | --- | --- |
| Pictures taken by Canon EOS Digital Rebel at different high ISOs (APS-sized image sensor) Click on image for original size [warning: large files] |  |  |

Practical Considerations

- You obtain the best image quality by using the lowest ISO possible on your digital camera. If you want to ensure your digital camera always uses the lowest ISO setting, switch the ISO setting from the default "Auto ISO" (this setting is usually found in the Menu) to the lowest possible on the camera, say ISO 50.

- If you mostly take pictures where there is enough light for a correct exposure, i.e. sunny outdoors, then using the lowest ISO on your digital camera will give you the best image quality your digital camera is capable of.

- If you want to take pictures indoors where light may not be sufficient and in other low-light situations, then you would need to supplement existing light with flash or studio lights. Either that, or select a higher ISO. Of course, depending on your digital camera, a higher ISO may mean a noisy image.

- A large image sensor (APS-sized and larger) means that you are able to use a high ISO speed without unduly worrying about noise. This means that you can take pictures in low-light situations without your pictures being under-exposed. It also means that in situations where it is required, you are able to use a fast enough shutter speed to prevent camera shake. All without noise.

Should You Use A High ISO?

Until consumer digital cameras start adopting the larger image sensors, noise will continue to be an ever present fact of life at high ISOs.

Another category of digital cameras for advanced amateur photographers -- commonly known as "prosumers" (professional consumers) -- attempt to bridge the gap between consumer and professional digital cameras by using a slightly larger image sensor (at 2/3 in. or 8.8x6.6mm).

However, the "megapixels race" has meant that ever more pixels are being crammed into a small area. Where before there were 5 million pixels on a 2/3 in. image sensor, now we see 8 million pixels crammed on the same sized image sensor. It is therefore not surprising that noise remains a problem. And which is why you should not be fooled by the "more megapixels is better" mantra.

A little bit of noise may not be a problem depending on the size of your prints or images for display. There are also a number of noise reduction software (Noise Ninja, Neat Image) that you can use to clean up the noise, though there's quite a bit of post-processing work involved, and you might want to reserve this for the special pictures you want to print large format.

If it is a matter of choosing between not being able to take a picture and suffering a noisy image, I'd rather be able to take the picture at a high ISO and then try to clean up the noise afterwards in a noise reduction software.

But remember, to be able to do this, your digital camera must allow you to select a high ISO (ISO 400, ISO 800). Some entry-level digital cameras have only one ISO, usually ISO 64 or ISO 100.

Is There A dSLR In Your Life?

If you definitely must take noise-free low-light pictures and therefore need to be able to use a high ISO speed, then you need to consider digital SLRs with their large image sensors. Just remember, though, that a dSLR usually requires a higher level of proficiency in using the camera and adjusting exposure than a point-and-shoot consumer digital camera does.

Recap

- ISO (equivalent) speed on a digital camera gives an indication of the sensitivity of the image sensor.

- The best image quality is usually obtained at the lowest ISO setting on your digital camera.

- If by adjusting the shutter speed / aperture combinations you cannot obtain a correctly exposed picture (usually in low-light situations), then you may want to select the next higher ISO.

- However, remember that using a higher ISO usually results in noisy images on consumer digital cameras.

- Use a high ISO if it is a choice between missing a picture and being able to capture an image -- even if it means you need to spend time cleaning out the noise in post-processing using a noise reduction software.

- If you leave your camera on "Auto ISO" and if you find that most of your images are noisy, then perhaps you are taking most of your pictures in low-light situations where the camera has automatically selected a higher ISO.

- If you are printing mostly 4x6 in., you may find the noise is not evident in the prints (and so you don't need to worry about noise at high ISO) though they may be visible at original sizes on screen.

# Understanding White Balance

If you come from the world of films, you may remember using filters to correct for incandescent or fluorescent lighting. Most people don't bother and their indoors pictures invariably come out with a yellow/orange or bluish cast. In the digital world, these correction filters are no longer necessary, replaced by a feature found in most -- even the entry-level -- digital cameras called, "White Balance."

Light Colour Temperature

The reason that pictures turn out with a yellow/orange cast in incandescent (tungsten) lighting and bluish in fluorescent lighting is because light has a colour temperature. A low colour temperature shifts light toward the red; a high colour temperature shifts light toward the blue. Different light sources emit light at different colour temperatures, and thus the colour cast.

By using an orange or blue filter, we absorb the orange and blue light to correct for the "imbalance" -- the net effect is a shift in the colour temperature.

In digital photography, we can simply tell the image sensor to do that colour shift for us. But how do we know in which direction of the colour temperature to shift, and by how much?

Manual White Balance

This is where the concept of "White Balance" comes in. If we can tell the camera which object in the room is white and supposed to come out white in the picture, the camera can calculate the difference between the current colour temperature of that object and the correct colour temperature of a white object. And then shift all colours by that difference.

Most advanced digital cameras therefore provide the feature to manually set the white balance.

By pointing the camera at a white (or gray) card, filling the screen completely with it, then pressing the White Balance button (or set it in the menu), the camera does its WB calculation.

From then on, any picture taken will have its colour temperature shifted appropriately. It's quite simple, really, and you should not be afraid to try it out and see your indoors pictures improve considerably (assuming there is enough light for correct exposure).

Preset White Balance

To help us in those special situations without having to go through the trouble of manually setting the white balance everytime, cameras provide preset WB settings such as, Tungsten, Flourescent, Cloudy, Sunny, etc. Using preset WB can improve on a picture, especially under indoors lighting.

| White Balance |  |
| --- | --- |
| AWB | Preset WB = Tungsten |

In the above example, the picture on the left is taken with the camera set to Auto WB. The indoor lighting is by two ordinary incandescent (tungsten) bulbs from the ceiling. It's not bad, but the fan should really be white. By dialing in a preset Tungsten WB, the image gives a truer representation of the scene.

Usually just selecting a Preset WB setting appropriate for the lighting situation is enough.

Auto White Balance

Since the days of the Kodak Brownie cameras, manufacturers have tried to automate everything for us. Hence, today's digital cameras also all sport an Auto White Balance (AWB) function. Depending on the camera brand, some AWB works better than others. On the whole, though, AWB works very well in sunny and cloudy outdoors, and fine for most indoors situations (a little orange or bluish cast does sometimes contribute to the mood of the picture anyway, e.g. the warm orangle glow of a candle).

Set your digital camera to AWB and take pictures under tungsten, fluorescent, and mixed lighting (i.e. tungsten or fluorescent, plus natural light coming through the window), and see if the results are OK. If they are, you can just use AWB.

Special Effects

Once you'se selected a WB setting, just remember to reset this white balance setting when you head back outdoors into natural light, or you may end up with some strange, out of this world, colours.

In fact, by dialing in a WB setting inappropriate for the lighting situation allows us to create some special effects.

Let's say you are taking a picture outdoors and you want to make the light warmer, perhaps creating a late evening, sunset effect. To do that, we dial in a Fluorescent WB, in effect telling the camera that the light is too cool. The camera responds by shifting every colour toward the warm, red values. Dial in a Tungsten WB, and the camera shifts all colours toward the cool, blue values. Lots of trial and error recommended here.

RAW File Format

A discussion in WB would not be complete without a mention of the RAW file format available in many advanced digital cameras. When you save an image in RAW file format, you are saving it the way the image sensor sees it -- without applying any adjustments (including white balance) to it. In fact, the camera ignores any WB setting you dial in.

Later, in an image editing software with the appropriate RAW plug-in, you can convert the RAW image to JPEG, and apply any colour temperature shift. Undo your change and try again, ad infinitum, in as fine an increment as you wish, until you obtain perfect colour balance.

Some professional photographers always use RAW file format. Saving in RAW file format comes at a price because it takes so much longer to save a RAW image that it might not be practical in many picture taking situations. Professional dSLRs (and some prosumer models) have internal buffers that allow RAW images to be taken one after the other in quite rapid succession without having to wait for the saving of one image to be completed before you can take the next picture.

If you are taking landscapes, and it's early in the morning or late in the evening, or you are not too sure of which WB setting to use, try it in RAW.

A few digital cameras even allow you to save an image in both RAW and JPEG simultaneously, though time to write to memory card is proportionally increased.

Summary

For most of us, Auto WB is fine and does a pretty good job in diverse situations, outdoors and indoors.

For those occasions when you are not happy with the colour cast of your picture, choose one of the preset WB settings.

If this still does not give you what you want, consider manually setting the white balance by using a white card or sheet of paper (or white T-Shirt, etc.).

And, for ultimate control over white balance, consider shooting in RAW file format, and adjust in post-processing.

# Depth of Field

What is depth of field, what affects depth of field, and how can we use an understanding of depth of field to take better pictures? This One Pager™ Tutorial will make everything clear from near to far.

What Is Depth of Field (DOF)?

Depth of field (DOF) is the distance wherein objects are in focus.

This is the pragmatic explanation.

To be technically correct, DOF is the zone of acceptable sharpness, the area in front of, and behind, a focused subject that appears in focus.

Circle of Confusion (COF)

Technically, only the subject in focus -- and all other objects at the same distance -- are in focus; everything else in front and behind are out of focus. How much out of focus depends on a term called the "Circle of Confusion (COF, or COC)."

Let's see if we can make that term clear, but if you don't understand it, don't worry. Many photographers take superb pictures using depth of field to their advantage never having heard of COF. I never did until I started researching this topic. And, I find that it just confuses the hell out of most people, so if you want to skip to the next paragraph, here is your chance to do so.

OK, back to the COF.

Imagine we are photographing three (3) dots. They are the tiniest dots the human eye can clearly make out, and of course, we are assuming perfect 20-20 vision and ideal light condition.

So, here we have these 3 dots, arranged one behind the other (with the closest dot to the right of the middle dot, and the farthest dot to the left of the middle dot, so a camera can take a picture of all 3 dots).

Now, we focus our lens on the middle dot, which comes out in perfect focus. The two other dots also appear in focus, but peering closely at the resultant photograph, we notice, however, that the dot in front of and behind the middle dot appears as circles instead of perfect dots. I.e., technically, they are out of focus, but to our naked eyes (at a "normal" distance), they "appear" in focus.

It is this circle that we call the "Circle of Confusion." So the COF is the diameter of a dot such that when we view it with the naked eye, it appears in focus. If this circle gets past this diameter, our eyes tell us it's out of focus.

Lens manufacturers have to decide what that diameter is going to be and design their lenses accordingly to be able to resolve a dot within that COF so that it appears sharp to us. For a more technically detailed explanation of how COF is determined by lens manufacturers, read Michael Reichmann's excellent tutorial: Understanding Depth of Field.

That is about all I am going to say on COF. Do you need to understand COF to use DOF? Personally, I don't think so. But if you are one of the people who absolutely must understand COF, its history, how lens manufacturers calculate DOF based on the COF, etc. etc., just check out the link above or do a search on Google. There's enough material there to keep a technical mind happy for many hours.

What Affects DOF

What we really want to know as photographers is what affects DOF so we can control DOF in our pictures. For a long, long time, photographers have gone with the following three criteria:

- lens aperture

- distance from subject

- focal length

While the first two are technically correct, the third one has raised somewhat of a storm of controversy among certain circles. Why exactly, we will make clear later. Let's look at each of the three criteria in more detail.

Lens Aperture

The aperture is simply the size of the opening that allows light to go through the lens. It is expressed in f/stops (also referred to as f/value or aperture value), and a typical aperture range is f/2.8 - f/8, giving the range from maximum (large at f/2.8) to minimum (small at f/8) aperture.

A small f/value (e.g. f/2.8) indicates a large aperture.

A large f/value (e.g. f/8) indicates a small aperture.

So, f/2.8 is a larger aperture than f/8.

Generally, a large aperture gives a shallow DOF, and a small aperture gives great DOF.

| f/2.8 | f/3.5 | f/4 | f/5.6 | f/8 |
| --- | --- | --- | --- | --- |
| large / max aperture |  |  |  | small / min aperture |
| less DOF |  |  |  | more DOF |
| great for portraits |  |  |  | great for landscapes |

Putting Aperture into practice: If you want only the subject the lens focuses on to be sharp, and everything else to be out of focus -- such as a portrait with the background nicely blurred -- then you would "open up the aperture," i.e. use a large aperture.

If you need most of your picture to be in sharp focus -- such as a landscape scene -- then you would "stop down the aperture," i.e. use a small aperture.

| Shallow DOF | Great DOF |
| --- | --- |
| Note how the use of a large aperture throws the flowers in the background out of focus. Focus has to be precise. | Using a small aperture extends the DOF from the foreground all the way to the background. |
| 49.8 mm, Av, Spot, 1/30 sec., f/3.5, +0.7EV, Macro, Tripod used | 49.6mm, Av, Spot, 1/5 sec., f/11, +0.7EV, Macro, Tripod used |
| Minolta DiMAGE A2 |  |

In most consumer digital cameras, you may not be able to directly control the aperture. Many, however, provide a Portrait scene mode and a Landscape scene mode that basically do what we are after, i.e. use a large and small aperture, respectively.

As most of you have found out by now, it is quite difficult to obtain a shallow depth of field with most consumer digital cameras even with the aperture opened up wide. Why? See the section on focal length below.

Distance From Subject

When you focus on a subject close to the camera, the DOF is less than when you focus on the subject farther away from the camera.

Putting Distance From Subject into practice: Step away from your subject to obtain greater DOF, or move in closer to decrease DOF.

Moving In Close:
Even though we use a small focal length (35mm, 35mm equivalent) and
a small aperture (f/7), by moving in close (macro), 
we are able to isolate the subject from the background.
If we had used a larger aperture, we would have obtained even shallower DOF.
Fujifilm FinePix S7000

Focal Length

Set your subject in front (say, about 3m or more) of a bush (or a tree with lots of leaves, or some other kind of busy background). Use wide-angle and take a picture. Both your subject and the bush would most probably be in sharp focus.

The background may distract from your main subject -- unless you're aiming for a Product Shot where you want to situate your subject in his or her environment, and do want both subject and background to be in sharp focus.

Now zoom in and fill in the screen with your subject's face and shoulders. Look at the resulting image and you'll probably notice that, though your subject is still in focus, the bush now appears out of focus, giving a nice blurred background that does not steal attention from your subject.

Photographers use this technique very effectively to "affect" DOF.

So, a wide-angle lens has greater DOF than a telephoto lens. Most consumer digital cameras have very short focal lengths and that is why it is so difficult to obtain shallow DOF, even with the aperture opened up wide.

I promised to come back to the storm of controversy brewing in some circles that disputes that focal length has any effect on DOF.

The contention is that, for a fixed image size in your screen, the DOF is unchanged irrespective of focal length used. And they have pictures to prove it!

However, photography is part technics and part art. The technics part may well tell us that DOF is the same at all focal lengths for a fixed image size on screen. What this is saying is that we achieve the same DOF whether we fill the screen with the face by walking to the subject, or backing off and zooming in.

The art side, however, tells us that a long lens (i.e. long focal length) reduces the distance from the subject and thus provides a shallow DOF. Yes, we can achieve the same narrow DOF by walking close to the subject. However, the results look very different! Why? Because a long lens also provides a "flattened" perspective which makes the narrow depth of field much more prominent than walking closer to the subject using a lens with normal focal length does. The link above with all the pictures prove it.

Putting Focal Length into practice: Use long focal lengths to achieve the effect of a shallow DOF.

Long Focal Length:
Bu using a long focal length (380mm, 35mm equivalent), we are able to 
"throw the background nicely out of focus"
Minolta DiMAGE Z1

Putting It All Together

Putting it all into practice:
A large aperture (f/3.2), moving in close to your subject (macro) and using a long focal length (112mm, 35mm equivalent) has thrown the grass into a nice green blur
Olympus Camedia C-8080 Wide Zoom

A Word On Aperture

Following our rule of thumb on using a small aperture to obtain a great DOF, does this mean that we should always use the minimum aperture if we want the greatest DOF?

It sounds logical, but because of diffraction that comes into play when the aperture is too small, this may affect the quality of the image, giving the appearance that some objects (especially light sources) are not in focus (but hey, use this to your advantage to obtain starry effects when photographing light sources!). So you don't have to stop down to the smallest aperture for maximum DOF. For example, on 35mm cameras (where the aperture stops all the way down to f/11 or even f/16), f/5.6-f/8 are usually the best apertures to use for maximum DOF. On digital cameras, try f/5.6.

Pseudo DOF

A word of caution. Some use post-processing and apply a Gaussian (or other type of) Blur to achieve a blurred background effect. Depending on your skills, this may be quite effective but, to a trained eye, this is almost always apparent and looks "false." So never try to pawn off a processed blur as naturally obtained shallow DOF. Also, be careful not to apply this blur effect to all your pictures, but only to a few whose subject matter will benefit from it and where it won't matter that the blur is processed, not natural.

Summary

Even though the subject you focus on is the only thing technically in focus, some objects in front and behind your focused subject also appear to our eyes to be acceptably in focus (thanks to the COF). This zone of acceptable sharpness is the DOF.

We can increase the DOF (or the appearance of it) by using a small aperture, moving away from our subject, or using a wide-angle lens.

We can decrease the DOF (or the appearance of it) by using a large aperture, moving closer to our subject, or using a long focal length.

# Night Photography

Night photography has an attraction all its own. There's something about scintillating lights from office windows hanging in the dark of the night -- a modern version of the starry skies -- that appeal to us. Whether it's a city skyline, lamp posts on a dark and deserted street, or the front of your house all decked out with holiday lights, the challenge of capturing the mood of a night scene depends on whether your digital camera is capable of night photography and on a couple of simple techniques.

Can My Camera Do Night Photography?

For successful night photography you need a digital camera that allows you to keep the shutter open for a long time, anywhere from 3 to 30 seconds. Check your camera specifications in the User's Manual under Shutter Speed. The shutter speeds available will be given as a range, e.g. 30 sec. - 1/2,000 sec.

If you are seriously into night photography, then you would want to ensure you purchase a digital camera that allows the longest shutter speed possible, and even Bulb (where the shutter remains open as long as you depress the shutter release button).

But before you plunk down your money for that digital camera, there's two more features to verify -- and one accessory to purchase, if you don't have it already.

Shooting Modes

For an image to be captured by a digital camera's image sensor, the latter requires exposure to light. But at night, light is what we don't have enough of.

Some of you may have noticed that, if you select a shooting mode of Auto (A) or Program Auto (P), your night pictures always come out too dark. They are simply underexposed. But, why is that -- if your camera's shutter speed ranges from, say 10 sec. to 1/2,000 sec.?

Go back to your camera's User's Manual and look a bit more carefully. Are all the shutter speeds available in Auto or P mode? Ah-ha, many digital cameras (we're talking consumer models here) do not make the whole shutter speed range available in A and P mode! Perhaps the slowest shutter speed available in A and P mode is only as slow as 1/3 sec. That's usually not long enough for night photography. To access the longer shutter speeds, you may need to select one of the other shooting modes, e.g. Shutter-Priority, or even switch to full Manual mode.

So ensure that your digital camera has full Manual mode and allows access to the full range of slow shutter speeds in that mode.

Self-Timer & Remote Controller

Another feature that you want your digital camera to have is a self-timer or, ideally, a remote controller. The purpose is to allow you to depress the shutter release button without introducing camera shake. I particularly like the remote controller, but not every camera comes with one or even has one available optionally.

But almost all, if not all, cameras has a self-timer. Usually the self-timer counts down from 10 sec. I find that a bit long to wait, especially since you would need to take more than one shot and it's minus 10 with the wind chill outside. The cameras that additionally provide a 2 sec. self-timer have my nod of approval here.

Tripod

A mandatory accessory that you need is a sturdy tripod. When you let the shutter stay open for a long time, the camera needs to be kept rock steady, otherwise you end up with blurred images.

Examples

OK, so we have our digital camera and tripod, and are ready to venture forth into the night in search for interesting night shots. When we find one, we set up camera and tripod, frame and... what do we do now?

Well, the images below show what happens when you take the same shot using P mode, then in Manual mode with various different shutter speed/aperture combinations, all in search of the correct exposure. The camera was on a tripod for all three shots.

Programmed Auto Mode - Underexposed

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 1/4 sec., Aperture F2.8, ISO 80

In the above example, the camera uses the slowest shutter speed and largest aperture available in P mode and at the widest focal length. The picture is underexposed.

Manual Mode - Overexposed

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 3 sec., Aperture F2.8, ISO 80

Switching to Manual mode allows me to access the slowest shutter speed available on this camera, 3 sec. while keeping the aperture at F2.8 (the largest aperture available). The effect is immediately better, but it does seem a bit too bright, giving almost a daylight effect. If this is the effect you're after, then you're done. But if you wanted to capture the night mood, read on.

Now it is just a matter of adjusting the shutter speed and/or aperture to obtain the desired exposure. I choose to close down the aperture so as to increase the depth of field also.

Manual Mode - Correct Exposure

Fujifilm FinePix E550 7.2mm, Programmed Auto, Pattern Shutter Speed 3 sec., Aperture F4.0, ISO 80

Closing down the aperture to F4.0, a more pleasant image is obtained with enough dark areas to indicate it is night time (dusk, really) and enough lighted areas to reproduce what my eyes saw at the outdoors skating rink at the Mississauga Civic Center.

ISO

For the three pictures above, I used ISO 80, the lowest ISO available for best image quality. But what if at 3 sec. and F2.8 (i.e. at max. exposure possible for this particular camera), the image still came out too dark? In this case, I would need to increase the sensitivity of the image sensor to a higher ISO. Do note that increasing the ISO also increases the amount of noise visible in your images.

Technique

Take a number of shots at different shutter speed/aperture combinations.

Immediately review the shot as soon as you've taken it.

Ensure your LCD brightness is set to Normal, not Bright, for a truer representation of your recorded image.

A good aperture to start with is F4.0 or F5.6 (for greatest depth of field), and adjust shutter speed up or down until you're satisfied with the shot.

For good measure, take an extra shot past your optimum exposure setting. For example, if you were progressively using longer shutter speeds, and you think you've find the correct one, take an extra shot with the next longer shutter speed. Conversely, if you were using progressively faster shutter speeds, take an extra shot using the next faster shutter speed.

# Understanding... Exposure Compensation

Most digital cameras provide exposure compensation. What is exposure compensation -- and how do we use it to obtain better exposed pictures? This OnePager™ tutorial makes it all clear.

The Short Description

Exposure Compensation is a feature of a camera that allows you to adjust the exposure measured by its light meter. Usually, the range of adjustment goes from +2 to -2 EV in 1/3 steps.

The Long Description

If you look at the QuickFact™ Sheet for a digital camera on this site, you'll notice this entry:

|  | Exposure Compensation | +/- 2 EV in 1/3 EV steps |  |
| --- | --- | --- | --- |

This means that you can adjust the exposure measured by the light meter by telling the camera to allow more light in (positive exposure compensation) or to allow less light in (negative exposure compensation).

Technically, you could take note of the exposure measured by the light meter, and then switch to Manual mode and adjust it manually yourself.

Depending on how your digital camera deals with exposure compensation (and the shooting mode used), it may adjust the aperture while maintaining the shutter speed constant, it may adjust the shutter speed while maintaining the aperture constant, or it may adjust both the aperture and shutter speed. On the digital camera I am using for this tutorial, the camera first adjusts the aperture while maintaining the shutter speed constant; when it can't adjust the aperture anymore, it then adjusts the shutter speed.

| Camera Metered Exposure |  |  |  |  |
| --- | --- | --- | --- | --- |
| F1.8 | F2.8 | F4 | F5.6 | F8 |
| 1/500 | 1/250 | 1/125 | 1/60 | 1/30 |

So, let's say that the light meter measures a shutter speed of 1/125 sec. and aperture of F4.0.

| +1EV Exposure Compensation |  |  |  |  |
| --- | --- | --- | --- | --- |
| F1.8 | F2.8 | F4 | F5.6 | F8 |
| 1/500 | 1/250 | 1/125 | 1/60 | 1/30 |

When I dial in a +1EV exposure compensation, the meter opens up the aperture by 1 f-stop to F2.8, while still maintaining a shutter speed of 1/125 sec. In effect I have purposefully dialed in an overexposure.

| -1EV Exposure Compensation |  |  |  |  |
| --- | --- | --- | --- | --- |
| F1.8 | F2.8 | F4 | F5.6 | F8 |
| 1/500 | 1/250 | 1/125 | 1/60 | 1/30 |

If I dial in a -1EV exposure compensation, the meter closes down the aperture by 1 f-stop to F5.6, while maintaining the same 1/125 sec. shutter speed. In effect I have purposefully dialed in an underexposure.

Exposure Bracketing

Why would anyone want to dial in an over or under exposure on purpose?

Because there are certain situations where the light meter of your digital camera can be fooled.

As an example, say you are taking a scene where there is an abundance of light around your main subject (for example, at the beach on a sunny day, or surrounded by snow). In this case, using Weighted-Average metering, your camera might be 'deceived' by the abundance of light and expose for it by closing down the aperture and/or using a faster shuter speed (assuming ISO is constant), with the result that the main subject is under-exposed. By dialing in a positive exposure compensation, you are making sure that your main subject is correctly exposed -- though the surroundings would be overexposed.

Another example would be the case where the surrounding might be too dark, and the camera exposes for the lack of light by either opening up the aperture and/or using a slower shutter speed (assuming ISO is constant), then the main subject is over-exposed. By dialing in a negative exposure compensation, you would in fact be under-exposing the surroundings, but properly exposing the main subject.

These are only two of the many situations where dialing in a positive or negative exposure compensation makes sense. In fact, many photographers will bracket their exposure when taking an important shot by taking one shot using the camera's measured exposure, a second one at positive exposure compensation, and a third one at negative exposure compensation.

Many digital cameras now include Automatic Exposure Bracketing (AEB) where the camera will take three shots with one press of the shutter release button: one at the camera measured exposure, a second at a negative exposure compensation (usually -1/3 EV, though some cameras allow you to specify the amount), and a third at a positive exposure compensation (usually +1/3 EV, though some cameras allow you to specify the amount).

Positive Exposure Compensation

In this example, the camera used a slower shutter speed while maintaining the aperture constant.

Camera Metered Exposure
Aperture = F4.0, Shutter Speed = 1/400 sec.
The bright sky "fools" the meter and the trees come out a bit dark

Exposure Compensation: +1.0 EV
Aperture = F4.0, Shutter Speed = 1/200 sec.
By applying a positive exposure compensation, the trees now show details

Negative Exposure Compensation

In this example, the camera used a faster shutter speed while maintaining the aperture constant.

Camera Metered Exposure
Aperture = F3.4, Shutter Speed = 1/340 sec.
The dark surroundings "fools" the meter and the sunset colours appear washed out

Exposure Compensation: -0.7 EV
Aperture = F3.4, Shutter Speed = 1/400 sec.
Applying a negative exposure compensation brings out the rich colours of the sunset

# Watercolor Effects

Creating a watercolor effect in Photoshop Elements is as easy as 1-2-3. This effect works best on a landscape photograph with wide swatches of colors.

First, we copy the image into Photoshop format before we apply the effect:

- We do a Select - All to copy the jpeg to the computer's memory.

- We then do a File - New... - OK to create an empty canvas with the same dimensions.

- An Edit - Paste will paste the image previously copied into the computer's memory onto our new canvas.

- File - Save this new picture with a new name as a photoshop file (.psd).

Next, if necessary, you might want to brighten up any areas that are too dark, or they will translate into solid blacks:

- Enhance - Brightness/Contrast - Brightness/Contrast... - Select Preview (if not already set ON). Then, slide the Brightness control right or left until the desired brightness is achieved.

- You can also select part of the image with the Marquee Tool and apply brightness only to that area.

- I darkened the highlights and brightened the shadows.

If you want to reduce the size of the image, do:

- Image - Resize - Image Size... - Select Constrain Proportions (if not already set ON). Type in your new Pixel Dimensions.

- I resized to a Width of 450 and the Height was automatically adjusted to keep the proportion.

- You may wish to crop the picture anyway you want.

Apply the Watercolor Effect:

- The watercolor effect is obtained at Filter - Artistic - Watercolor...

- In the example below, I used a Brush Detail of 14, Shadow Intensity of 0, and Texture of 2.

- Depending on your image, you might want to play with those numbers until the effect pleases you.

Save your newly created watercolor image as a new file:

- File - Save As... and save it as a Photoshop file.

- If desired, convert it back into a jpeg for posting to the web or for emailing (give it a new file name). Some resolution will be lost in the compression: File - Save for Web... and save as a JPEG with Medium compression.

Here is another example:

# Add A Border

Adding a border around a picture can sometimes be an effective way to highlight the picture. Instead of going automatically with borders that look like traditional picture frames, consider more original and elegant options. Today's image editing software usually provide a number of interesting options that you can customize to fit the mood of the picture.

Canon PowerShot S50
Night Scene mode, Evaluative, B&W
10.3mm, 1/4 sec., F3.5, ISO Auto

In this One-Pager™ tutorial, we'll explore how to create a border for the picture above. I want to add a thick black border all around it, but don't want it to be solid. Even though, we could just select one of the many Photoshop border actions available free on the Internet, we'll instead walk you step by step in creating one so you understand how those actions are created -- and you can then create your own. We will use Photoshop Elements and make the assumption you are somewhat familiar with it.

The picture is in jpeg, so the first thing to do is to open it up in Photoshop Elements and then Select - All, followed by Edit - Copy. This copies the picture into memory (or the clipboard, if you want to be more exact). Close the original picture since we never ever want to muck around with it.

Now, we open up a new file to work on: File - New... - Contents=Transparent - OK, followed by Edit - Paste. The picture is pasted unto our new file.

[Of course, though we are specifying the menu bar options (in bold), feel free to use the icon shortcuts instead.]

Save this file as a Photoshop file (.psd) with a new name. It's a good idea to save your work each time you reach a milestone -- or before you try a major change: File - Save - give it a name [e.g. chess_border.psd].

Since we want to add a border around it, we need some space around it first: Image - Resize - Canvas Size.... The image is currently 400W x 300H pixels. I enter 450W x 350H to create a 25 pixels border all round (you may choose whatever border thickness you wish to work with).

If your Layers window is not opened yet, click on the Layers tab and drag it into the main window. It will now stay open. If necessary, drag the bottom edge down to be able to see more than one layer at a time.

There will be a Layer 1 already in the Layers window with the picture we're working on. Double-Click on that layer and rename it to something more appropriate, such as "Original".

Now, create a new layer by clicking on the "Create a new Layer" icon at the bottom of the Layers window. A new layer (again called Layer 1) appears above the Original layer. Double-Click on Layer 1 and rename it "Background". Drag the Background layer below the Original layer. You should end up with something like this:

I always create a Background Layer separate from other layers. Later you may fill this layer with a background color or just leave it transparent.

Click on the Original Layer, then create a new layer. Rename it to "Border".

With the Border layers still selected, use the Rectangular Marquee Tool to delienate the picture. Then inverse the selection to select everything outside the picture: Select - Inverse. Fill this selection with Black color.

At this point we have a solid black border 25 pixels thick all around the picture. I really want the black border to be only 10 pixels thick, so I use the Rectangular Marquee Tool to delienate a rectangle around the picture 10 pixels thick (use the ruler markings to guide you). I then inverse the selection again to select everything outside of both the picture and the 10 pixels thick border. I delete the inverted selection, leaving a clean 10 pixels black border around the picture.

I want this black border to be hatched, and Photoshop Elements have the Strokes Frame effect that will do just that.

With the Border layer still selected, use the Magic Wand Tool to select the border only: just click on the Magic Wand Tool (If your Tools palette is not showing: Window - Show Tools) then click on the border. Marching ants around the border show that the border has been selected.

Apply the Strokes Frame effect: Click on the Effects tab - scroll down to Strokes Frame and click on it to select it - click Apply.

Flatten Layers? Click OK.

Do you wish to keep this Effect? Click Yes.

And there you have it! Experiment with the different effects and filters Photoshop Elements provides. If you make any mistake along the way, Edit - Undo or Step Backward will safely undo your mistake. Have fun!

# Isolate Your Main Subject

Sometimes a picture just has too many places for the eyes to focus on. On a trip to Canada's Wonderland, I took a picture of the lifeguard watching the children (including mine) having a blast on the water slides. In modifying this photo, I had two goals in mind: 1) I wanted to isolate the lifeguard and focus the attention on her alone, plus 2) I wanted to give the impression of swirling motion (for that is really how it felt).

Sony DSC-V1 Cyber-shot
Program AE, Multi Pattern, Auto WB
7mm, 1/250 sec., F5.0, +0.7EV, ISO 100

In this One-Pager™ tutorial, we'll explore how to use the Radial Blur action of Photoshop Elements to accomplish both goals.

Work in Photoshop Format

The picture is in jpeg, so the first thing to do is to open it up in Photoshop Elements and then Select - All, followed by Edit - Copy. This copies the picture into memory (or the clipboard, if you want to be more exact). Close the original picture since we never ever want to muck around with it.

Now, we open up a new file to work on: File - New... - Contents=Transparent - OK, followed by Edit - Paste. The picture is pasted unto our new file.

[Of course, though we are specifying the menu bar options (in bold), feel free to use the icon shortcuts instead.]

Save this file as a Photoshop file (.psd) with a new name. It's a good idea to save your work each time you reach a milestone -- or before you try a major change: File - Save - give it a name [e.g. watchfulCare.psd].

Work on a Copy

If your Layers window is not opened yet, click on the Layers Tab and drag it into the main window. It will now stay open. If necessary, drag the bottom edge down to be able to see more than one layer at a time.

There will be a Layer 1 already in the Layers window with the picture we're working on. Double-Click on that layer and rename it to something more appropriate, such as "original".

Now, make a copy of this layer by dragging it to the new layer icon (the middle icon) at the bottom of the Layers window. A new layer (called original copy) appears above the original layer. Click off the eye icon beside the original layer. You should end up with something like this:

Select Area To Keep In Focus

Now comes the tricky part. First, ensure you are working with the copy by selecting it (click on the original copy layer). Using the Lasso Tool and Magic Wand Tool, trace the outline of the lifeguard and any other part of the picture you want to stay in focus.

If you make any mistake along the way, Edit - Undo or Step Backward will safely undo your mistake(s). Also, remember that you still have the original layer if you mess up real good and want to start all over. For example, if you want to delete the original copy layer because you messed it up, position your cursor on the layer and do: Right Click - Delete Layer. Then make a copy as described above.

Once you have selected the part of the image you want to keep in focus, do Select - Inverse to inverse the selection since we want to apply Radial Blur to that part.

[Of course, you can opt to do the reverse: select the area to blur. Up to you, and depending on which area is smallest and/or easiest to select.]

Apply Radial Blur

Now, do Filter - Blur - Radial Blur...

Experiment on Amount, Blur Method, and Quality to achieve the effect you want. Click OK.

And there you have it! The radial blur amplifies the swirling action of the children sliding down and running back for one more go at the water slides, all under the watchful care of the lifeguard in sharp focus. Try the Radial Blur action of Photoshop Elements. Have fun!

Photoshop Elements Radial Blur Effect

# Add A Signature

You've posted your digital images on your web site or a public photo hosting site, and thinking, "Wouldn't it be nice if I could add a signature on all my images?" Here is one easy way to do this in Photoshop Elements.

What Are Layers?

Photoshop Elements has the concept of "layers" that sounds complicated but really isn't.

You can think of a layer as simply being a transparent page.

Unlike a real-life transparent page that does mask the original image beneath a little, the digital version is 100% transparent.

Just like illustrators use transparent pages to add shadows on one layer, colours on another, text on a third, etc. -- so the original image is untouched, and each graphic expert does not muck up the work of the other -- Photoshop layers allow you to do the same.

So, that's great, because this means that you can do all your image editing on one or more layers (as many layers as you need) without touching the original image. And if you make a mistake, just chuck out that layer and start with a fresh one. Want to experiment on different text effects, create a layer for each effect, and then hide or display them one by one to see which one you like the best.

How do we create a layer?

You can do so thru the menu (Layer - New - Layer...) or, if you have the Layers Palette already open (simply drag the Layers tab to your work space), click on the "Create a new layer" icon (highlighted in red below).

To select a layer to work on, simply click on that layer in the Layers Palette. To delete a layer, Right-Click on that layer in the Layers Palette and select Delete Layer - Yes.

Ok, so now that we have the basics of creating and removing layers out of the way, let's create our signature layer that we will then use on our images.

# Add A Signature

cont.

Create A New File

Open Photoshop Elements and do File - New...

Enter a width and height, and leave the other entries as is. You may also enter a Name (as I did here) for the file or do it later when you save the file.

Note that it is important to set Contents to "Transparent" because you want your signature to float over your image.

At this point, all we have is a new file with a transparent background (represented by the white and gray checkers).

I find it is quite difficult to see anything drawn on a transparent background, and prefer a white background to work with. So let's create a background layer and fill it with white.

# Add A Signature

cont.

Create A Background Layer

Still remember how to create a new layer. Simply click on the "Create a new layer" icon.

Your Layers Palette should end up looking like this:

Layer 1 was there when we created a new file. It is transparent.

We just created Layer 2, and it is also transparent. Layer 2 is highlighted, i.e. it is selected. If it is not selected, click on it to select it.

On your Tools Palette (if it is not displayed already, do: Windows - Show Tools), set your foreground colour to White. It usually is by default. If it is not, then click on it and select white.

Tools Palette
Currently, the foreground colour is set to White
and the background colour is set to Black.

Drag the mouse to the upper leftmost of the screen,
or type "FFFFFF" (six Fs, upper or lower case) into the "#" edit field.
"FFFFFF" is the hexadecimal equivalent of "White."
Click OK to accept the colour as your foreground colour.

Now, click on the Paint Bucket Tool on the Tools Palette, and when you move your mouse over your drawing canvas, the mouse will change into a paint bucket. Click on the drawing canvas to colour the canvas entirely in white.

I like to name my layers so I know what they are later. So, I will rename Layer 2 (that we just coloured all white) to "background" and the way to do this is to Double-Click on Layer 2 in the Layers Palette and type in the new name in the Layer Properties.

Layer 2 will be renamed background in the Layers Palette.

You can, of course, choose any colour you want to use as a background. So, if you want to sign in black, then a white background is appropriate. If you want to sign in white, then select a black background. This background layer is simply to allow us to see clearly what we are going to type next.

# Add A Signature

cont.

Type In Your Signature

To type in your signature, select the Horizontal Type Tool on the Tools Palette.

Click on your drawing canvas and type your signature:

Photoshop automatically creates a new layer for the text type.

If you look at the top of your screen, you will notice that the Font face, type and size are displayed. Click on the drop-down lists and select your desired font. Usually we would choose a small font size so the signature does not intrude too much into the picture, but for this tutorial I have selected Verdana at 24pt so we can see it clearly:

If you need to change the font, simply click on the Type Tool again, highlight the text, and change the font:

I changed the Font face and size to Staccato222 BT 36pt.

Your Layers Palette should end up looking like this:

# Add A Signature

cont.

Save The Signature

When we are happy with the font face, type and size of our signature, all we have left to do is to save the file for future use. But before we do this, we want to restore the transparent background and hide the white background.

To do this, we unselect the background layer by clicking on the eye symbol beside it so that it disappears. Notice that the transparent background is restored.

Click on File - Save on the menu and save your signature file.

That's it!

Now everytime you need a signature, you just open up your signature file, copy it to the clipboard, and paste it into a new layer on your image.

Let's do it right now while we still have the signature file open.

To copy the signature, do Select - All, Edit - Copy Merged. Your signature is now saved in the clipboard.

Now let's add the signature to an image.

# Add A Signature

cont.

Add Signature To Image

Here is the image I want to add my signature to:

I paste (Edit - Paste) my signature on the image, and Photoshop automatically pastes it into a new layer. Or, if you want to be safe, click on the "Create a new layer" icon to create a new layer, and then paste your signature into that new layer.

Photoshop pastes the signature in the center of the image. Select the Move Tool on the Tools Palette, then click and drag the signature to the bottom right (or, wherever you want it to be) of the image.

Oops, I've got a problem. Since I selected to type my signature in black, it can be barely seen against a dark background.

Despair not, friends, for we shall remedy to this situation illico presto!

We could go back to our signature file and change the colour of the text to white. Nay, that's too easy, and besides, we want to learn a few more Photoshop tricks.

- Add A Signature

cont.

Create Cool Semi-Transparent Layer

First, let's rename the Layer 1 to signature. To do this, Double-Click on Layer 1 on the Layer Palette, and type in signature, click OK to accept the name change.

I am now going to add a new layer underneath the signature layer. The idea is to then fill that new layer with a light colour so the signature can be read clearly.

Click on the background layer, then click on the "Create a new layer" icon to create a new layer above the background layer but underneath the signature layer. [If necessary, you can drag and drop layers up and down one another.] Since we had renamed the previous Layer 1 to "signature," Photoshop creates a new layer and names it Layer 1 again.

Select the Rectangular Marquee Tool on the Tools Palette, then draw a rectangle around your signature, being careful to leave equal amount of space between the left and right sides, as well as the top and bottom sides. You can reposition the marquee rectangle with your mouse or using the arrow keys. Using the Paint Bucket Tool , fill this rectangle with a white colour (or any colour you want).

Well, the signature is clearer now, but the white background is distracting and looks pretty unprofessional. I know, let's give it one of those semi-transparent treatment that looks so cool.

To do that is child's play. See the "Opacity" drop-down on the Layers Palette? Simply change the percentage to one that gives you the right result.

By default, the layer with the white paint (Layer 1) is 100% opaque, i.e. we cannot see through the white paint to the picture underneath. Click on the Opacity drop-down and slide the the lever left and right until the effect looks just right to you. I settle on an opacity of 34%.

And, here is the image with the signature:

There, you have it. Pretty simple, huh?

And, here are a few more example signatures:

| White Text | Medium Outline | Simple Outer Glow |
| --- | --- | --- |